Contents:

1. Terms and definitions	3

2. General principles	5

3. Employees	13

4. Road and Pedestrian Traffic Principles	15

5. Equipment brought to the premises of ArcelorMittal Poland S.A.	18

6. High risk works	19

7. High Risk Lifting Operations	25

8. Fire-related aspects and   high-risk plant	28

9. Accidents and first aid	36

10. Use of machines, equipment and tools owned by ArcelorMittal Poland S.A.	37

11. Environmental protection	37

12. Inspections conducted in the place of work	38

13. Contractual penalties	38

15. Declaration	39

List of enclosures:

- Template of the Passport for persons working for ArcelorMittal Poland S.A.

- Booklet for Contractors.

- Template HIRA Risk Assessment Form

- High-risk lifting operations plan for cranes.

- Authorization for employees of external companies

- FPS Standards Booklet.

- Health and Safety Alert - Preliminary information about the accident.

- Template of near miss/risk of a near miss report.

- Catalogue of fines

- List of consequences

# 1. Terms and definitions

1.1. 	"Contractors' Safety Manual" or the "Manual": a document attached to the Agreement, being a compilation of fundamental rules that a Contractor working for ArcelorMittal Poland S.A. is obliged to follow. It does not release the Contractor from an obligation to observe regulations of law and provisions of other documents and guidelines.

1.2. 	"Passport" – an internal document issued by the Company which confirms that the document holder:

  - has a valid medical check-up certificate stating that they are fit to work or to perform specific operations,

  - has all legally required qualifications to perform specific work or operation,

  - passed a test verifying knowledge of basic H&S principles effective in ArcelorMittal Poland S.A.

  - works for a specific Contractor.

Passport issued by the Company is a basis for issuing a pass allowing the Passport holder to enter and perform work on the Premises of ArcelorMittal Poland S.A.  The passport is valid only with an ID document with a photograph. The Passport holder must have it on them at all times and present it at the request of the Company's employees. Passport Template is included in Enclosure no.1.

1.3. 	Premises of ArcelorMittal Poland S.A. or Company's Premises – real property, to which ArcelorMittal Poland S.A. holds the legal title or where it actually runs its business, in particular areas and land enclosed by fence (including but not limited to ground-based, overground and underground structures, roads and yards, buildings, warehouses and production bays, railway sidings and truck loading yards).

1.4. 	"Work or Work on Premises of ArcelorMittal Poland S.A.": repair, modernization, maintenance, demolition, investment, loading or unloading, as well as any other services/ operations performed by the Contractor based on the "Contract for Performance of Work on ArcelorMittal Poland S.A. Premises”. This also applies to services/ operations and works performed by Subcontractors.

1.5.    	"Contractor": executing the Contract for Performance of Work on Premises of ArcelorMittal Poland S.A. - a contractor, service provider, supplier of goods, performer of a specific task, an entity executing construction, assembly, demolition works and similar services and activities on premises of ArcelorMittal Poland S.A. Provisions of the Manual concerning the Contractor are also applicable to all Contractor's and  Subcontractor's Employees unless this Manual expressly states otherwise.

1.6.	"Subcontractor": an entity subcontracted by the Contractor to execute the Contract for Performance of Work on ArcelorMittal Poland S.A. Premises, in particular: a contractor, service provider, an entity executing construction, assembly and demolition work.

1.8.	"Contractor's Employee": a natural person, engaged by the Contractor to execute Works on the Premises of ArcelorMittal Poland S.A., including an employee in the meaning of the labor law regulations and a person employed (engaged) by the Contractor on any other legal basis. An Employee should also be understood as a natural person, employed (engaged) by any other entity, working for the Contractor based on a contract with a third party (in particular interim employees).

1.9.	"Subcontractor's Employee": natural person employed (engaged) under any legal grounds by the Subcontractor, who performs works/activities or services on the Premises of ArcelorMittal Poland S.A. Whenever the Contract or the Manual refer to Contractor's obligations towards their Employees, the same obligations apply to Sub-contractor's Employees unless the Contract or the Manual state otherwise.

1.10.	"Employee”: Contractor's and Subcontractor's Employee jointly.

1.11. 	OH&S rules and regulations: regulations concerning occupational health and safety established by universally effective provisions of the law, internal regulations of the Company and in this Agreement which may be made more specific and supplemented by the Health and Safety Office of ArcelorMittal Poland S.A. H&S regulations and principles may be subject to change.

1.12. 	"Agreement" or "Agreement on Occupational Health and Safety: Agreement on Occupational Health and Safety together with all appendices, including the Contractor's Safety Manual and appendices to the Manual.

1.13.	Contract for Performance of Work on ArcelorMittal Poland S.A. Premises: a contract concluded by and between ArcelorMittal Poland S.A. and the Contractor (either in a written form or by sending documents by fax or e-mail or otherwise), by virtue of which the Contractor is obliged to perform Work on the Premises of ArcelorMittal Poland S.A.

1.14.		"FPS Standards”:  fatality prevention standards: ST001 "Isolation”, ST002 "Confined Spaces”, ST003 "Work at Height”, ST004 "Rail Safety”, ST005 "Shop floor Audits”, ST006 "Vehicles & Driving ”, ST007 "Cranes & Lifting”, ST008 "Contractors Management”, ST009 "Alert”, ST010 "Safety Metrics”, ST011 "Incident Investigation"”, ST012 "Working in Gas Hazard Areas”,   ST013 "Emergency Preparedness & Response”, ST014 "Hazard Identification and Risk Assessment”.

1.15.	"Safety Supervisors of the Contractor”: Contractor's employee nominated to supervise compliance with the safety, fire protection and environmental rules during the performance of Work on the Premises of ArcelorMittal Poland S.A. Supervisors are nominated individually to supervise performance of specific works. The supervisor should be indicated by name in the Contract for Performance of Works on the Premises of ArcelorMittal Poland S.A., or if the Contract for Performance of Works on the Premises of ArcelorMittal Poland S.A. has been concluded in a form other than in writing, or when completing single orders under such a contract - in a separate document  at the stage of approving a Contractor to perform work on the Premises of ArcelorMittal Poland S.A. Work manager may also fulfill the role of the Supervisor. Appointing the supervisor of safety compliance on the part of the Contractor does not release the respective Contractors and Subcontractors from the obligation to provide healthy and safe working conditions for employees.

1.16.	"Persons responsible on the part of ArcelorMittal Poland SA"

- Safety Coordinator: an employee of ArcelorMittal Poland S.A. supervising work health and safety of all employees working in a given area.

- Representative of ArcelorMittal Poland S.A. (for example Repair Manager, Investment Manager, etc.) an employee of ArcelorMittal Poland S.A. authorized to directly contact a Contractor with respect to execution of work under the Contract for Work Performance on ArcelorMittal Poland S.A. Premises who permits the Contractor to start work and participates in the works acceptance.

- Health and Safety Office: ArcelorMittal Poland S.A. organizational unit responsible in particular for Health and Safety issues,

1.17. 	HIRA: Identification of actual or potential risks and assessment

of occupational risk for works performed or planned.

1.18. 	"Facility Handover Form": document signed by representatives of ArcelorMittal Poland S.A. and the Contractor prior to the commencement of Works on the Premises of ArcelorMittal Poland S.A. The Facility Hand-over Protocol is enclosed to the effective instruction on the performance of tasks in the maintenance area in ArcelorMittal Poland S.A.

1.19 	Authorization for employees of external companies: an internal document of ArcelorMittal Poland S.A. confirming that a person holding this document has been licensed by the Technical Supervision Office or Transportation Technical Supervision Office to operate/maintain crane equipment. All licenses granted by the Technical Supervision Office/Transportation Technical Supervision Office should be listed in the "Authorization..." The document should be filled in legibly and approved by the superior of the external company employee. An employee of an external company should keep the "Authorization..." visible (Appendix no. 5).

1.20. 	"H&S Observer” : an employee appointed to monitor and supervise works as regards safety, and intervention aimed at eliminating risk or accident risk.

# 2. General principles

The objective of ArcelorMittal Poland S.A. is to eliminate incidents that may lead to accidents and occupational diseases in accordance with OH&S Policy of ArcelorMittal Poland S.A.

Cooperation between all employees performing work on premises and for ArcelorMittal Poland S.A. (internal employees, Employees of Contractors, Subcontractors, staffing agencies and interim employees), is the prerequisite for achieving this objective.

The main task of this Manual is to specify the minimum requirements that employees must fulfill to achieve the objective of ArcelorMittal Poland S.A.

Contractors are fully responsible for their employees. They are also responsible for employees of their Subcontractors and interim workers who must demonstrate their knowledge and understanding of the regulations and act in accordance with this Manual.

It is prohibited to hire employees in a way that does not comply with labor law regulations.

Contractors and Subcontractors are not allowed to hire an employee of ArcelorMittal Poland S.A., whatever the legal basis, to perform works on the premises of ArcelorMittal Poland S.A. or for ArcelorMittal Poland S.A. If such a case is discovered, the employing entity bears the full responsibility, including responsibility defined by this Agreement, and is obligated to reimburse all costs, charges and fines (including social and health insurance contributions), imposed on ArcelorMittal Poland S.A. by state authorities in connection with discovering such employment.

In case of hiring natural persons performing work on a basis other than employment relationship and persons running their own business, Contractors must ensure safe  and healthy conditions for execution of work and ordered tasks, as well as for any trainings and courses organized on plant premises for students who are not the employees of their company. Whenever works are performed in a place accessible to persons who do not participate in work or fulfillment of the Contract for the Performance of Works on the Premises of ArcelorMittal Poland S.A., Contractors are obliged to apply the necessary measures to protect their health and life.

In particular, Contractors are responsible for safety conditions during task execution.

Employer's scope of responsibility is independent of the employees' obligations related to 
 occupational health and safety and assignment of OH&S function tasks to the external specialists mentioned in Art. 237¹¹ § 2 of the Labor Code.

Contractors must protect employees' health and life by ensuring safe and healthy working conditions through appropriate use of scientific and technical achievements. In particular, Contractors are obliged to:

1) 	organize work and task execution in a way ensuring safe 
 	and healthy working conditions,

2)	ensure that OH&S regulations and principles are followed; give instructions for elimination of any irregularities in this scope and follow-up on the fulfillment of such instructions,

3)  respond to needs related to ensuring occupational health and safety and take measures to improve the existing level of employees' health and life protection, taking into account the changeable conditions of work performance or task execution,

4)  ensure development of a consistent policy to prevent accidents at work and

occupational diseases, allowing for technical aspects, work organization, working conditions, social relations and impact of working environment,

5) take into account protection of the health of minors, pregnant or breastfeeding

mothers, as well as disabled employees in scope of preventive measures taken,

6)	ensure fulfillment of orders, requests, decisions and directives issued by bodies that

supervise working conditions,

7)	ensure fulfillment of recommendations issued by a social labor inspector.

8) Prior to commencing Work on the Premises of ArcelorMittal Poland S.A.  a list of people who they will hire to perform the Contract Work, and shall ensure that they underwent all the required trainings on OH&S issues (preliminary, vocational, periodic - refers to trainings organized by the employer for their employee) should be submitted to ArcelorMittal Poland S.A. The list shall be updated. Prior to the commencement of works, the persons in question should be provided with a pass authorizing them to enter the premises of ArcelorMittal Poland S.A. (in line with the Contractors' Safety Manual).

Contractors and employee supervisors must be familiar with work safety regulations, including OH&S rules and regulations in the scope necessary to perform their duties.

Contractors must define their own OH&S principles and regulations and ensure that they are observed in their companies. Contractors must meet the requirements specified in this Manual. ArcelorMittal Poland S.A. may request the Contractors to go through verification audits, which the Contractors have to accept.

During work performance, it is prohibited to expose oneself or others to the risk of injury or damage to property of ArcelorMittal Poland S.A.

Contractor's employees are obliged to immediately inform their supervisors and representatives of ArcelorMittal Poland S.A. about identified accidents, near misses and environmental hazards, as well as risk to property, and Contractors are obliged to immediately inform ArcelorMittal Poland S.A. thereof.

In case of failure to observe OH&S regulations and rules, ArcelorMittal Poland S.A, reserves the right to discontinue works, expel Contractor's employees from ArcelorMittal Poland S.A. premises, impose regulatory or contractual penalties, and as a last resort, terminate the Contract for the performance of Work on the Premises of ArcelorMittal Poland S.A. and hold the Contractor liable for the damage that occurred.

Contractors are obliged to observe health and safety, fire protection, environmental protection regulations, regulations concerning energy/optimal consumption of energy, and regulations on the protection of people and property and trade secrets effective on premises of ArcelorMittal Poland S.A.

- 2.1. Contractors' competence.

Before a company is selected to work on premises of ArcelorMittal Poland S.A. Contractors must undergo the process of preliminary qualification. The qualification process includes the assessment of the Contractor's technical capacity in scope of OH&S requirements.

ArcelorMittal Poland S.A. evaluates Contractors' performance in terms of implementing the provisions of the Safety Manual in accordance with the adopted internal procedure at least once a year.

ArcelorMittal Poland S.A. reserves the right to employ an external auditor to conduct an audit at the Contractor's. The purpose of the audit is to verify reliability of information related to work organization and observance of H&S regulations and principles provided by the Contractor. The audit may be conducted in the location where the Contractor runs their business (seat/plant/premises). Contractor will be informed of such an audit in advance.

- 2.2. Selection of the contractor:

The Contractor's documents submitted in connection with tender must include OH&S actions, which cover all risks regarding health and safety, as well as the method of their elimination. Safety Plan assessment will be one of the criteria for the Contractor selection in the tendering procedure.

- 2.3 Subcontracting

Any Contractor hiring a Subcontractor or planning to hire a Subcontractor is obligated to obtain the approval of ArcelorMittal Poland S.A. to do so. Each Subcontractor must undergo the same process of preliminary qualification and selection in scope of safety.

Contractor is obligated to inform their subcontractors about all rules and regulations related to occupational health and safety and environment protection and other defined under the general principles of this Manual. Contractor is obligated to inform their Subcontractors about all arrangements and requirements related to the performance of a specific Work on the Premises of ArcelorMittal Poland S.A.

Contractor is responsible for the observance of occupational health and safety rules, environmental protection regulations, and internal regulations of ArcelorMittal Poland S.A. in the aforementioned scope, both by their employees and those of Subcontractors, and their Subcontractors.

- 2.4. Contractor Categories:

- A- category: covers Contractors who take over the site owned by ArcelorMittal Poland S.A. to perform work requiring building permit as understood in the Building Law (e.g. "Turn-key" investments);

- B- category: covers Contractors, whose scope of work on premises of ArcelorMittal Poland S.A. is specified by ArcelorMittal Poland S.A., which issues the permit to work and the work is to be performed in the period longer than 30 days (including Contracts for Performance of Works on the Premises of ArcelorMittal Poland S.A., concluded for a period of one year -  the so-called Annual contracts)

- C- category: covers Contractors who will perform work on premises of ArcelorMittal Poland S.A., work duration will not exceed 30 days and ArcelorMittal Poland S.A. employees will be involved.

- D- category: covers Contractors who will perform work outside the production facilities of ArcelorMittal Poland S.A. in the period not longer than 1 day (including computer hardware servicing etc.) and Contractors providing Transport services (drivers), as well as people invited by Contractors (Visitors)

- 2.5. Obligations of Contractors covered by categories A, B & C:

It is the duty of Contractors covered by A, B & C categories to acquaint all their Employees, as well as Employees of Subcontractors with requirements included in the Manual against a written confirmation.

In case of selection of Contractors covered by categories:

- A: preference for companies with a certified quality management system based on ISO 9001 and/or ISO/TS 16949, occupational safety management system based on PN-N 18001 and/or OHSAS 18001, environment management system based on ISO 14001 and energy management system based on ISO 50001;

- B: preference for companies with an implemented quality management system based on ISO 9001 and/or ISO/TS 16949, occupational safety management system based on PN-N 18001 and/or OHSAS 18001, environment management system based on ISO 14001 and energy management system based on ISO 50001; The system does not have to be certified;

- C: preference for companies which have basic safety procedures in place (i.e. risk assessment, responding to breakdowns and work accidents)

In the case of companies holding SCC (Safety Certificate for Contactors) or certificate of ArcelorMittal Poland S.A., obligations resulting from the Manual are agreed individually. Health and Safety Director makes agreements on behalf of ArcelorMittal Poland S.A.

- 2.6. Other obligations of the Contractor:

- log on to "For Contractors" tab at poland.arcelormittal.com” immediately (within 1 working day) after receiving information that Work has been contracted (order), in order to sign up for a knowledge test.

- Prior to commencement of works, each employee of the Contractor must pass a knowledge test on the basic principles of occupational health and safety of ArcelorMittal Poland S.A. The test may be held once a Contractor trained their subordinate Employees and those of Subcontractors. The training may be provided by the H&S Office or the Contractor. If this is the case, the training should be based on materials available at poland.arcelormittal.com, the Tab "For Contractors" will have an application providing the Contractor with training materials once they have registered. The Contractor will be obligated to enter their Trained Employees or Subcontractor's Employees for an exam which will be held at the H&S Office in ArcelorMittal Poland S.A.  A positive result of the test will be confirmed by the Health and Safety Office of ArcelorMittal Poland S.A. A negative result will mean that the permit to perform Work on the Premises of ArcelorMittal Poland S.A.  by a given Employee of the Contractor or Subcontractor will not be granted.

- All Contractor Employees who did not complete the aforementioned training must be informed about basic threats listed in relevant leaflets before they enter premises of ArcelorMittal Poland S.A. Premises may only be entered under the supervision of an employee of ArcelorMittal Poland S.A.,

- Contractor's Employee is obligated to inform the H&S Coordinator or the repair manager about devices subject to technical inspection that are brought to the premises of ArcelorMittal Poland S.A., with details specified in the Facility Hand-over Protocol,

- employees of transport companies (drivers) must be equipped with footwear with steel toe-caps, three or four point chin strap helmet, protective glasses, hearing protectors and reflective vest,

- all Contractor and Subcontractor's Employees must analyze in detail all manuals and instructions given, respect them and confirm in writing that they understood them,

Contractor's obligations:

- train their Employees who will perform Works on the Premises of ArcelorMittal Poland S.A. on internal regulations effective in ArcelorMittal Poland S.A. applicable during the perfomance of works.

- prepare equipment, tools, dangerous substances and personal protection equipment to be inspected by AMP representatives, and prepare a list confirming that all tools that will be used are in a proper technical condition,

- indicate, in the construction log, a person responsible for permanent actual supervision over Contractor's and Subcontractor's Employees involved in Works on the Premises of ArcelorMittal Poland S.A., and a person responsible for checking vehicles and access,

- participate in works (device, facility) handover and acceptance,

- use electrical power machines and equipment which fulfill requirements arising from effective regulations on the construction and operation of electrical power equipment,

- participate in meetings devoted to Works on the Premises of ArcelorMittal Poland S.A.

- fence locations where Work is performed on the premises of ArcelorMittal Poland S.A. and put up signs required by effective regulations and agreements,

- secure materials entrusted against damage and theft,

- respect health and safety and fire protection regulations and keep the area where works are performed in order,

- together with AMP representative prepare a HIRA Risk Assessment Form (Appendix no. 3), including information on required safety measures and risks related to neighboring installations, machines and devices, potential interruptions if Work is performed on the Premises of ArcelorMittal Poland S.A. at the same time and area by more than one Contractor or Subcontractor.

- train Contractor's and Subcontractor's Employees on existing risks and procedures to be followed in case of emergencies (training completion should be confirmed with a relevant entry in the Construction/Works Log), with detailed information on aspects covered by the training and safe behavior and performance of works with respect  to neighboring installations,

- obtain permits allowing Work on the Premises of ArcelorMittal Poland S.A. as agreed,

- make a detailed settlement of materials collected from AMP representative once Work on the Premises of ArcelorMittal Poland S.A. has been completed,

- respect regulations on environment protection, and in particular have a permit for waste generation and a procedure for waste disposal,

- ensure that Contractor's and Subcontractor's Employees have valid licenses enabling them to perform work. When hiring a Subcontractor, the Contractor is obligated to make an entry in the works log and specify the name of the Subcontractor, names of the Subcontractor's employees and an appointed works coordinator.

- provide Contractor's and Subcontractor's Employees with valid training and medical certificates, qualifications required for the performance of contracted works and who have Passports.  Documents confirming entries in the Passport must be available for verification and control at the request of AMP representative,

- make correct and detailed entries in the documentation related to the Work performed on the Premises of ArcelorMittal Poland S.A. (in particular: Construction log, Works log, written instructions on works performance and other documents required for a given Work on the premises of ArcelorMittal Poland S.A.), including dates on which entries are made and a legible signature,

- provide permanent actual supervision over Contractor's and Subcontractor's Employees in the location where Work is performed on the Premises of ArcelorMittal Poland S.A.

by a person appointed in writing (name specified). Permanent actual supervision means presence and contact with Contractor's and Subcontractor's Employees by an appointed Employee of the Contractor in the location where work is performed for as long as the works continue

- ensure that Contractor's supervisors conduct audits to check if H&S regulations and principles are observed, the audits are to be confirmed with a relevant entry in the Construction/works log during works performance and after their completion.

- provide a continuous and actual supervision by Safety Team (meeting the requirements specified in the relevant regulations), in their own scope and at their own expense, in the following situations:

  - when the Contractor performs a part of or entire investment - classified as such by ArcelorMittal Poland (constant daily supervision),

  - when the Contractor performs exceptionally risky works (principles of supervision agreed in the Facility Hand-Over Form prior to commencing the works), always during demolition works

  - when the employee of H&S Office of ArcelorMittal Poland requests constant supervision.

The detailed principles of supervision and any amendments to the aforesaid arrangements must be included in the Facility Hand-over Protocol

- properly document scrap handed over,

- prepare and provide the repair manager with a list of other waste,

clear the area and restore it to its original condition once the works are finished.

- 2.7. Work Permit.

Prior to commencing Work on the Premises of ArcelorMittal Poland S.A., Contractor's employees should be acquainted with documents effective for a given area, i.e.

- Requirements of the Integrated Management System effective in ArcelorMittal Poland S.A. applicable to the area where Work is performed on the Premises of ArcelorMittal Poland S.A,

- "Register of significant environmental aspects",

- "List of potential breakdown sites",

- "Operational management sheet for significant energy consumption",

- "Operational control significant risks sheet",

- Rules and requirements resulting from fatality prevention standards effective in ArcelorMittal Poland S.A. related to the location and character of works performed.

Before the task specified in a relevant contract or order is executed, the Safety Coordinator is obliged to hold a meeting with a Contractor to discuss safety issues and work organization. Facility Hand-over Protocol must be prepared after the meeting.

In case of any changes in the situation that could influence the conditions of work health and safety, ArcelorMittal Poland S.A. and Contractor are obliged to hold a meeting again to adjust the previously agreed OH&S action plan to the risks present.

Prior to commencing Work on the Premises of ArcelorMittal Poland S.A., AMP representative appointed by a manager of ArcelorMittal Poland organizational unit, together with a Contractor assess the occupational risk (Appendix no. 3 to this Manual - HIRA assessment form), that may occur during or in connection with performing Work on the Premises of ArcelorMittal Poland S.A. related to the contracted task, with special focus on:

- gas-hazard zones,

- ionizing radiation,

- technology applied in the work site, counter-measures and protection equipment as well as detailed OH&S instructions.

- explosion hazard zones.

- technological risks (production processes, machines, equipment),

- rules of safe crossing of the plant (for example walkways to work stations),

- warning and evacuation rules.

Identification of risks and occupational risk assessment is to be made jointly in the location where works are performed.

Meetings on organization of safe work must be held periodically, 
and always in case of changes that have influence on working conditions and safety.

Necessary conditions to start Work on the Premises of ArcelorMittal Poland S.A.:

- Contractor must have relevant documents in accordance with principles 
specifying Contractor's tasks and duties effective in ArcelorMittal Poland S.A.;

- H&S plan defining the manner and order in which works should be performed must be prepared and in place.

- Contractor must have a filled out and signed Facility Hand-over Protocol which should be submitted to the Health and Safety Office of ArcelorMittal Poland S.A. 3 days prior to the commencement of works at the latest,

- Contractor must have a HIRA form (template in Appendix no. 3) prepared and approved by the works manager and AMP representative for risks existing on the site where works are performed or in connection with works performed,

- Contractor must have documents confirming that Contractor's and Subcontractor's employees were acquainted with  the aforesaid documents, particularly in terms of risk assessment for the works performed,

- Contractor must have documents confirming that Contractor's and Subcontractor's Employees have been acquainted with rules and requirements of fatality prevention standards effective in ArcelorMittal Poland S.A. related to the site and character of the works performed.

- Contractor must provide their employees with work and protective clothing (with a company logo), footwear with steel toe-caps and with PPE (protective glasses, three or four-point chin strap helmet, etc.) and a reflective vest.

Each Contractor’s Employee staying on Premises of ArcelorMittal Poland S.A. must wear a reflective vest. If the working/protective clothes have large and visible reflective elements, the Contractor's employees do not have to wear the reflective vest.

Clothes in the same colors as those used in ArcelorMittal Poland S.A. are strictly forbidden. Clothes must clearly show which employees are Contractor's Employees and which are ArcelorMittal Poland S.A. employees and must bear the company's logo.

In special conditions, such as breakdown removal, preventing a disaster, etc. the Contractor is not obliged to draw up the aforesaid documentation prior to the commencement of Work on the Premises of ArcelorMittal Poland S.A. The only exception is a prepared HIRA (template in Appendix no. 3). The required documentation should be completed within the period of 48h from the moment of starting Work on the Premises of ArcelorMittal Poland S.A. If there is a breakdown or an accident during performance of Work on the Premises of ArcelorMittal Poland S.A., further works should be performed under special supervision. Such a procedure is permissible only if there is a direct threat to employees' life or health, or a risk of losing high-value property to provide safety. A Plant Director appoints a supervisor (coordinator) to supervise further works performed by the Contractor after HIRA has been prepared (by an appointed team), to define conditions facilitating safe completion of this work.

The documentation, specified above, must be drawn up in at least two copies. One set of documents is kept by the Contractor in the location where Work is performed on the Premises of ArcelorMittal Poland S.A. and is available at the request of internal and external control services. The other set of documents is kept by AMP representative.

Contractor will not let their employees start work without personal protection equipment required at a given work station and without LOTO padlocks required if there is a necessity to isolate hazardous energy.

Black LOTO padlocks for Contractor's and Subcontractor's employees are provided by ArcelorMittal Poland S.A.

During the works performance, the Contractor is obliged to conduct them in a safe way, taking into account the activity of other Contractors and ArcelorMittal Poland S.A. employees.

When works are discontinued by an authorized employee of ArcelorMittal Poland, they can be resumed only based on a permit of task performance by ArcelorMittal Poland Representative following consultations with a person who discontinued works.

The Contractor performing works in the explosion hazard zone is obliged to provide employees with anti-static work wear. In the explosion hazard zone it is forbidden to use electrical equipment and tools other than those approved for "Ex" zones.

The equipment, tools, containers, cylinders, vehicles, ladders, scaffoldings etc. used must correspond to effective regulations and cannot pose a threat to other employees. Gas cylinders must be stored only in places properly adjusted for this purpose and identified in consultation with ArcelorMittal Poland S.A. Representative.

Contractor's Employees cannot move around the premises of ArcelorMittal Poland SA without a permit. Each entry or exit from the work site must be reported to the work/ repair manager or any other employee designated by ArcelorMittal Poland S.A. (e.g.: dispatcher)

Contractor performing work in explosion hazard zones must acquaint their employees, prior to starting work, with safe work principles resulting from risk assessment for explosion hazard zones.

The fact of informing employees of the aforesaid principles must be documented and the work in the explosion hazard zone must be regularly supervised by a person responsible on the part of the Contractor.

In production bays and storage yards (marked zones) of ArcelorMittal Poland S.A., footwear with steel toe-caps, dust protection goggles, three or four-point chin strap helmets, protective clothes with reflective elements or reflective vest and other PPE must be worn in line with the markings used.

- 2.8. Principles of hiring other companies by the Contractor

The following conditions must be met by the Contractor when hiring other companies as their Subcontractors to perform Work on the Premises of ArcelorMittal Poland S.A.:

- must be agreed with the Procurement Office

- must ensure that requirements of the Agreement on health and safety are fulfilled,

Contractor is obliged to acquaint their subcontractors with all rules and regulations concerning OH&S, explosion and fire protection, as well as environmental regulations and any other regulations referred to in item 2 in the introduction.

Any deviations from the rules specified above require a joint written approval of a representative of the Procurement Office of ArcelorMittal Poland S.A. The said approval may not be understood as approval defined in art. 6471 § 3 of the Civil Code.

The obligation of reporting Subcontractors hired to the Procurement Office lies with the Contractor, who is a party to the Contract for Performance of Work on the Premises of ArcelorMittal Poland S.A. Failure to report a Subcontractor to the Procurement Office may result in a prohibition to perform work on ArcelorMittal Poland S.A. premises.

- 2.09. Confidentiality

Contractor undertakes to treat all information acquired during performance of works on ArcelorMittal Poland premises as a trade secret protected by the law as intangible asset of ArcelorMittal S.A. under the pain of liability for damages.

- 2.10. Housekeeping

Contractor is responsible for housekeeping on ArcelorMittal Poland S.A. premises, especially on the site where Work is performed on the Premises of ArcelorMittal Poland S.A. premises until Work on the Premises of ArcelorMittal Poland S.A. is completed and accepted.

# 3. Employees

- 3.1. Entering/leaving the premises of ArcelorMittal Poland S.A.

Each Employee of the Contractor and Subcontractor must have a name pass which authorizes them to enter Premises of ArcelorMittal Poland S.A.

When presenting a pass, Security Services have the right to request for an ID Card to be presented (with identification details and a photo). All information is included in the Pass System for people and vehicle traffic effective in ArcelorMittal Poland S.A.

The basis for applying for a pass authorizing entry on Premises of ArcelorMittal Poland S.A. site is a valid correctly filled out Passport.

The Contractor is obliged to collect the Passport form from the Health and Safety Office and fill it out correctly (template in Appendix no. 1).

The "Passport" must be filled out in a legible way, including:

- name of the Contractor for which the Contractor or Subcontractor's Employee works and telephone number in the company, at which information on the Employee of the Contractor or Subcontractor may be obtained,

- legible stamp with the name of the Contractor, telephone number,

- the stamp and signature of the occupational medicine physician admitting to work or certifying the capacity to perform work at a given position 
 and under particular conditions must be obtained. The stamp and signature must be obtained during periodical check-ups or other examinations. Possible contraindications/permits to perform specific works, e.g. work at heights, should be specified, if applicable. In the absence of notes referred to above, it is assumed that an employee cannot perform work at height.

NOTE! If works are performed by a foreigner, the medical certificate   should be translated and the person supervising works is obligated to write a statement that the translation is true and employees are fit to perform work.  The aforementioned documents must be attached to the passport.

- qualifications of a Contractor's or Subcontractor's Employee confirmed with relevant documents are filled out by the superior, confirming the above with their legible signature.

- confirm a positive result of the test checking  knowledge of basic H&S rules effective in ArcelorMittal Poland S.A. organized by the Health and Safety Office of ArcelorMittal Poland S.A. - to be done by an employee of the H&S Office of ArcelorMittal Poland S.A. who confirms that with their own signature that, except for Zdzieszowice Unit where a superior confirms the positive result of the test in a Contractor's or Subcontractor's Employee Passport after the Employee's personal data has been entered to the electronic record in the H&S Office and after the entry pass has been obtained.

H&S training and the knowledge test organized by the H&S Office are valid for the same period as the results of medical tests confirmed with a doctor's stamp in a given Passport (2 doctor stamps possible on one form). When the passport is changed because medical tests are no longer valid (based on a doctor’s second stamp), it is necessary to complete a new H&S training. When there is a request to "rewrite" the confirmation of training completion in a new passport e.g. due to employer's change, only trainings held less than one year from the date on which such a request is made may be rewritten. Rewritten training completion confirmation must be confirmed by the employee's signature on the attendance list, and the positive result of the final test. A Passport that lost its validity is taken over by the H&S office.

The training and test in Zdzieszowice Unit must be completed periodically, every three years

Taking photographs/ video recording on ArcelorMittal Poland S.A. Premises is prohibited.

3.1.1. Exceptions from the obligation of holding the Passport.

The following parties are not obliged to hold the Passport:

- Employees of subsidiaries, where ArcelorMittal Poland S.A. holds 50% share or more, whereas such employees must strictly follow the rules provided for in the Manual and the Agreement. Such subsidiaries act on behalf of ArcelorMittal Poland S.A. in relation to their Contractors representing its position included in the Manual and Agreement (among others they collect passports for them, control their distribution to Contractor's employees),

- Employees providing one-day equipment servicing, performing works of technical emergency, maintenance works in office buildings, etc. on Premises of ArcelorMittal Poland S.A.

- Employees of entities providing people and property security services based on the contract concluded with the Company,

- Employees of the companies that do not directly Perform Work on the Premises of ArcelorMittal Poland S.A., who periodically control the works performed, e.g. Company president, their deputies, company owner (such information is sent by the company to the Health and Safety Office ).

- Employees of external bodies of control: National Labor Inspection, Sanepid (National Sanitary inspection), Office of Technical inspection, etc.,

- Persons responsible for housekeeping in the production bays and administration buildings, however, in such case the Facility Hand-over Protocol must be prepared and provided to the Health and Safety Office. This may be covered by the company registration for the entire period of work performance in a given facility in line with the contracted period (e.g. year, quarter, month) The persons in question must be acquainted with the risks present at a given worksite.

- Employees of companies performing other works, after a written release from the obligation of holding the Passport has been issued by the H&S Office. Such decisions are made by an appointed employee of the H&S Office.

- 3.2. Verification of authorizations, periodic trainings and medical examinations

Contractor’s and Subcontractor's Employees must have valid periodic trainings, medical checkups and authorizations adequate to actions taken in scope of the tasks to be completed. Detailed authorizations and qualifications are determined by the Polish regulations of the law.

H&S Office employees and the representative of ArcelorMittal Poland S.A. may require the Contractor to present the relevant documents. Documents for verification and control must be always available in the location where Works are performed.

Employees of H&S Office, ArcelorMittal Poland S.A. representatives and other ArcelorMittal Poland S.A. employees are authorized to verify and to discontinue works performance as a result of failure to observe H&S regulations and principles by Contractor and Subcontractor.

# 4. Road and Pedestrian Traffic Principles

- 4.1. Pedestrian traffic principles

Contractor's employees should only use:

- designated walkways for pedestrians;

- remain at the site where work is performed,

- places where Contractor's social facilities are located

It is forbidden for Contractors’ employees to be present in places where they do not perform the contracted work (in particular in the areas of production lines, warehouses, railway tracks and office rooms).

- 4.2. Vehicle traffic principles

Public road traffic regulations apply to ArcelorMittal Poland S.A. premises, with the following speed limits:

Kraków – 				40 km/h

Dąbrowa Górnicza –  			40 km/h

Sosnowiec – 				20 km/h

Świętochłowice: 			20 km/h

Chorzów – Królewska – 		20 km/h

Zdzieszowice – 	                       40 km/h

On premises of all ArcelorMittal Poland S.A. Units it is also admissible to use bicycles, however in such case, reflective vests must be worn at all times.

Drivers must unconditionally follow:

- road traffic regulations,

- road signs located within the premises of ArcelorMittal Poland S.A., concerning, among others, speed limits, right of way, no-traffic areas, STOP signs, etc.

- any instructions given by authorized employees of ArcelorMittal Poland S.A. responsible for traffic control.

Railway vehicles always have the right of way on ArcelorMittal Poland S.A. premises. This means that a driver must stop in front of each railway crossing (even the one without STOP sign).

It is forbidden to smoke in moving vehicles or vehicles used to perform works on premises of ArcelorMittal Poland SA.

The driver and passengers of the moving vehicles must have their safety belts fastened.

The use of mobile phones and other communication devices (e.g. walkie-talkies) is forbidden when driving vehicles.

Vehicles transporting hazardous substances must be marked in accordance 
with the effective regulations.

Security services and H&S Office are authorized to control the observance of road traffic regulations, in particular: breathalyzing of drivers, checks of validity of the vehicle’s technical inspection, safe parking rules, and measurement of vehicles’ speed, use of lights, and use of safety belts.

Failure to observe the above rules will result in the prohibition to enter the premises of ArcelorMittal Poland S.A., and in justified cases the offenders 
will be apprehended, with the application to start proceedings placed with the law enforcement bodies.

- 4.3. Vehicles parking

Vehicles may be parked in designated and marked car parks and in the so-called temporary car parks (designated and marked) to be used during repairs, investments etc.

Bays may be entered only for loading and/or unloading which is always supervised and performed on conditions defined by a "bay representative" who is an employee of ArcelorMittal Poland S.A.

The vehicles must be parked in a way that does not pose a risk for railway traffic (for example limited visibility), with particular attention to clearance gauge of tracks.

- 4.4. Keeping roads in a good condition

Heavy-duty machines used for construction without rubber or fixed tires (e.g. with metal tracks, road rollers, compactors with metal elements having contact with road surface) may be brought to ArcelorMittal Poland S.A. premises only on trailers towed by vehicles with rubber or fixed tires.

Contractor is responsible for immediate removal of any materials or goods accidentally left on roads on ArcelorMittal Poland S.A. premises. Such an area should be properly marked, and H&S and Corporate Governance and Property Management Office must be informed thereof.

All kinds of damage resulting from infringement of these measures will lead to compensation claim against the Contractor.

Road Traffic Law and relevant executive regulations apply to transport of over-sized and heavy cargo on premises of ArcelorMittal Poland S.A.

Transport of such kind of loads must be reported to the security services of a particular location of ArcelorMittal Poland S.A. and to the entity responsible for road management used for this particular transport.

The report should include information on transport route, type of load, date and approximate transport time.

For each transport of over-sized and heavy cargo, the carrier should prepare a HIRA (Enclosure no.3).

- 4.5. Maneuvers in low visibility conditions

When a driver of a vehicle must reverse in low visibility conditions, it is required that another person be present to support them with voice instructions or using signals. Any other people close to the truck must be informed about the plan to reverse the truck. To avoid any crushing risk, all necessary measures must be taken to ensure nobody is in the area between the maneuvering vehicle and the fixed obstacles.

All maneuvers must be performed very slowly.

While unloading a dump truck or a transport vehicle the same measures must be taken as it is the case for reversing in low visibility. It is obligatory while maneuvering a dump truck or unloading a transport vehicle that another person is present to provide support. Directing the driver must proceed in accordance with safety rules, at a safe distance from the vehicle, in order to avoid any risk of crushing. One should make sure that all warning, light and sound signals work properly and are not off during such maneuvers.

It is not allowed to start a maneuver unless all the requirements are met.

Vehicles, mechanical equipment and bicycles entering the premises of ArcelorMittal Poland S.A. must be technically operational and may not pose a risk to other traffic participants inside the company or to natural environment.

NOTE: hazardous materials transport within the premises of ArcelorMittal Poland S.A.  comes under strict regulations. Incoming hazardous material: hydrazine 40%, hydrochloric acid, sodium hydroxide, sodium hypochlorite, lubricating hardening oils, diesel oil, leaded petrol, engine oil, ammonia, oxygen, and others considered hazardous must be reported at the entry gate where detailed information about the route is given.

Failure to observe the effective regulations and limits is a basis to cancel the personal authorization to enter the premises of a given ArcelorMittal Poland S.A. Unit in a vehicle.

Corporate Governance and Property Management Office reserves the right to control the observance of road traffic regulations and to call the Police if they consider this necessary.

# 5. Equipment brought to the premises of ArcelorMittal Poland S.A.

- 5.1 Transporting material to and from plants

Entry and exit of Contractor’s trucks and other vehicles is possible only based on the permit granted, valid together with a pass in line with internal regulations of ArcelorMittal Poland S.A.

A material pass is required for transport of material and equipment to and from ArcelorMittal Poland S.A. premises.

It is prohibited to take or transport any material being the property of ArcelorMittal Poland S.A., or of another Contractor from ArcelorMittal Poland S.A. premises without a material pass.

Taking or transporting any material from ArcelorMittal Poland S.A. premises is admissible only based on a material pass. If such materials are to be brought to the premises, the above fact must be reported to a relevant representative of the Security Office as required by the regulations on material circulation and pedestrian and vehicle traffic in ArcelorMittal Poland S.A.

It is prohibited to bring any chemicals and hazardous materials, explosives, radioactive materials, not used for performance of the commissioned works. All such materials, brought in or out of ArcelorMittal Poland S.A. premises must have REACH material description sheets.

- 5.2. Material loading and unloading

- Drivers of transport companies are allowed to stay in the loading stations located in the bays only if they wear proper PPE, i.e.:  three or four-point chin strap helmet, reflective vest, footwear with steel toe-caps, dust protective glasses, uniform: long-sleeve shirt, trousers with long legs, individual hearing protection (stoppers, head phones, ear plugs) - if necessary due to exceeded maximum noise level (NDN).

- It is forbidden to remain on the truck's platform during material loading or unloading with crane equipment.

- Climbing and descending the truck platform is possible by using ladders or stairs only. Jumping down the platform is forbidden.

- While loading or unloading with crane equipment, the driver may not stay in the truck cab.

- When getting climbing down/out of/in the vehicle, the driver must have three points of contact.

- The driver is responsible for correct securing of the materials on the truck.

- Over-sized materials that protrude beyond the contour of the truck must be labeled in accordance with effective regulations.

- 5.3 Equipment coming under the inspection of Technical Supervision Office brought to ArcelorMittal Poland S.A. premises

Contractor bringing the equipment coming under the inspection of the Technical Supervision Office to the premises of ArcelorMittal Poland S.A. must consult the works manager/H&S coordinator, prepare the hand-over form and receive a relevant entry pass. It is necessary to hold an authenticated copy of the current decision of the Technical Supervision Office.

# 6. High risk works

- 6.1. Performance of dangerous works

Dangerous works are defined in Title IV Section 6 of the Regulation of the Minister of Labor and Social Policy of September 26, 1997 on general occupational health and safety conditions.

ArcelorMittal Poland S.A. representative will inform the Contractor on the types and methods of performing dangerous works on the premises of ArcelorMittal Poland S.A.

Dangerous works will be performed exclusively after Contractor receives a one-time written permit for the performance of a given work, if such is required on ArcelorMittal Poland S.A. premises. This permit is issued in accordance with regulations effective in ArcelorMittal Poland S.A.

Each dangerous energy or dangerous substance must be isolated and isolation itself must be checked before works are started on the equipment.

Contractor's and Subcontractor's employees are allowed to perform works requiring isolation of hazardous energy or substances on condition that:

- each of the aforementioned employees is provided with a black LOTO padlock

- each of the aforementioned employees is trained in procedures under FPS001 standard - ISOLATION

- the requirement to isolate hazardous energy or substances in specific cut-off points is observed by each of the aforementioned employees when they enter the area/site where repair works are performed.

Black LOTO padlocks for Contractor's and Subcontractor's Employees are provided by ArcelorMittal Poland S.A.

Dangerous energy:

  - Electrical energy,

  - Pneumatic energy,

  - Hydraulic energy,

  - Thermal energy,

  - Radiation,

  - Mechanical energy,

  - Chemical energy,

  - Acoustic energy

hazardous substances – substances and mixtures of substances which because of their chemical, biological or radioactive properties may pose a threat to human life and health or the natural environment if they are not handled correctly; a hazardous substance refers to raw materials, products, semi-finished product, waste and a substance generated as a result of a breakdown (e.g. flammable, caustic and toxic substances):

  - Solids,

  - Gases,

  - Vapors,

  - Fluids,

  - Dust.

Particular discipline must be kept when working in gas-hazard zones, and explosion-hazard zones, where performance of work must each time be consulted  with the Gas Surveillance Station relevant for a given location.

Works performed in zones at risk from explosive atmospheres and in gas-hazard zones may be carried out in line with safety requirements defined in effective safety procedures, policies and work organization procedures for gas installations and equipment specified in internal Company regulations and commonly binding law.

NOTE!

There are also high-risk works performed in ArcelorMittal Poland S.A. (as per HIRA). Should it be necessary to perform such works, a decision must be made jointly by a team (including the Plant Director, H&S Director and plant leader of FPS standard applicable to the works), to increase supervision and possibly use cameras. The solution adopted should be included in HIRA.

- 6.2 Marking the place of work and risk areas

The place of work performance by the Contractor must be marked in such a way that information is provided about potential or existing risks (e.g. boards informing about risks), and must be fenced (e.g. with a warning tape, safety railings). A Contractor is obligated to mark their work site in a proper way.

- 6.3. Works at height

Works at height are defined in Title IV Section 6 letter E of the Regulation of the Minister of Labor and Social Policy of September 26, 1997 on general occupational health and safety conditions.

Works at height on the Premises of ArcelorMittal Poland S.A., must be conducted in accordance with safety rules effective in ArcelorMittal Poland S.A. and generally effective legal regulations.

Performance of works on the roofs is allowed only after relevant "work performance permit" is obtained from the OH&S Coordinator of ArcelorMittal Poland S.A. The permit in question must include the list of risks present in a specific area, drawn up by the employee of ArcelorMittal Poland S.A., where the performance of works will take place.

A permit to perform works at height may be issued only after all the preventive measures confirmed in the permit card for work at heights have been performed, them being specifically:

- assessment of the technical condition of the roof in terms of roof covering strength 
and a possibility of safe works performance,

- permanent and visible marking of dangerous zones (e.g.: skylights, areas with reduced strength surface, etc.).

- preparation of HIRA Risk Assessment Forms for all the activities conducted in the place of work performance allowing for the risk of fall not only outside but also inside the bay or a building,

- presenting the approved plan of works performance, drawn up on the basis of HIRA Risk Assessment Forms, including: direct supervision over these works, instructions during the performance of particular activities, works distribution, sequence of tasks performance and use of efficient and certified fall protection equipment.

- information to all the employees with the assessment of risk - upon written confirmation,

- fencing and marking the works site in order to limit access,

- indicating and marking the "drop zones" from the roof (if necessary),

- designation of an employee by name to supervise the work of his/her subordinates on a continuous basis and to ensure that everybody is attached to life lines and anchoring points in a correct and safe manner.

Works at height must be performed by two persons at least and controlled regularly by a representative of the Contractor.

When performing works at height which require the use of personal protection means against fall (safety harness, ropes with shock absorbers, adjustable ropes with shock absorbers, anchor points, life lines etc.), the Contractor must provide constant supervision over Employees.

The supervision should be based on a monitoring system. The system is to facilitate live monitoring of the behavior of Contractor's and Subcontractor's Employees and should record in loop mode, at least 72 hours. Rules defining who will watch the material recorded and how often should be agreed with the Plant Director, separately for each work.

Any deviation from the above must be agreed with the Plant Director, representative of the H&S Office and the plant leader for FPS Standard "Work at Height". Approval of ArcelorMittal Poland Chief Operating Officer is required.

HIRA Risk Assessment for the work must include information on how the supervision will be provided (e.g. camera system or observers)

All Employees performing works at height, including persons supervising the works, must use certified fall protection equipment.

Each time before the works at height start, the Contractor is obligated to provide training to their Employees on how to inspect, put on and use individual fall protection equipment.

Employees can be transported upwards or lifted only by means of equipment intended for this purpose and meeting the required regulations.

WARNING:

Each employee performing work at height where, according to HIRA Risk Assessment, there is a risk of fall from height and protective measures include attaching oneself to anchoring points or life lines, MUST BE ATTACHED ALL THE TIME WHILE PERFORMING WORK.

Double life line principle:

Principles of using life lines

Principles of using shock-absorbing equipment

HIRA Risk Assessment for works at height should always specify the type of individual shock-absorbing equipment proper for the risk of a fall from height.

For example:

- Line without shock-absorber of the length ....... (height of fall below 4m)

- Line with a shock-absorber (height of fall above 4m).

- Self-locking device

- Line with a fall arrester

- Double life line

- Other (specify)

Contractors are obligated to use helmets with a three-point or four-point chin strap for all works.

Collective protection means such as safety nets under roofs should be used unless HIRA specifies that they are not required for the performance of a task.

Portable ladders used by Contractor's employees on AMP Premises must be:

- marked and labeled in a manner facilitating their identification

- checked by designated Employees of the Contractor periodically and each time before they are used. Periodical checks should be documented.

- 6.4. Work in confined space

Confined space is understood as an area where human movement is limited or more difficult for physical reasons and which has not been designed or intended as a constant place of work. The space may have limited access / exit (e.g. a manhole) and the following risks may be present there:

- atmosphere with agents harmful to health (gases, vapors),

- atmosphere with reduced oxygen content,

- entrapment.

Confined spaces refer to, among others to:

- tanks, silos, chambers, boiler vessels, pressure tanks, tank-like chambers - with a manhole, ceiling and floor (bottom) only, piping and cables (especially energy galleries),

- open-pit spaces or excavations more than 1.5m deep,

- pipes, pumps, sewage canals, drains, tunnels, areas under equipment and installations, underground facilities and similar structures,

- BOF vessel, reheating furnace chambers, where access/exit can be difficult or where oxygen-less atmosphere or gases or harmful vapors can occur.

Works in confined spaces may be performed only after a written permit has been obtained from authorized employees of ArcelorMittal Poland S.A. The permit will include the HIRA presented by ArcelorMittal Poland S.A. for a specific space (Appendix no. 3).

Any work in confined space must be preceded by risk assessment prepared by the Contractor, specifying the risks that may occur during works performance: Such as:

- lack of oxygen,

- presence of toxic gases,

- risk of explosion,

- risk of being buried or risk of drowning,

- fall from height,

- falling objects or tools,

- contact with products that are toxic, hazardous to life or aggressive,

- contact with products that are dangerous to human health or aggressive,

- other.

The assessment is to determine the equipment necessary to protect employees, and to specify the means of individual and collective protection.

The works in question must always be performed under the supervision of one or a few persons, who are all the time outside the risk zone and are equipped with communication devices in case it is necessary to call emergency.

The Contractor's employees must be instructed on the risk assessment
 related to work performed in a confined space. Such instruction is to be documented.

- 6.5. Works performed close to liquid metal

All employees working close to or in the vicinity of areas where liquid metal eruption may occur, must be properly trained. The additional training is conducted by a manager of an organizational unit of ArcelorMittal Poland S.A. or a person designated by them.

Contractor must provide their employees with suitable collective and personal protection equipment.

- 6.6. Electrical works

The Contractor may start works only when a person responsible for shutting down a machine in ArcelorMittal Poland SA releases suitable documentation or a permit for works specifying the type of works, date of works performance and detailed scope, said details to be specified in writing. According to the principles adopted in ArcelorMittal Poland S.A. a written order for electrical works is required.

A person responsible for shutting down the equipment in ArcelorMittal Poland SA is the only authority that may specify the technical and technological conditions making it possible to shut down a machine or put it out of operation. This person is also responsible for specifying the conditions of restarting or commissioning the equipment. The above principles do not apply to new facilities before the technical acceptance, and in special cases being the subject of a special contract.

The Contractor's employees operating the electrical equipment must have licenses required by law, must undergo a training in scope of actions to be taken  in case of risks related to electrical installations and hold written permits relevant to works performed.

Access to electrical installations and facilities is restricted to authorized electricians with national licenses only in accordance with effective regulations.

6.7 Demolition works

Contractor is obligated to define detailed occupational health and safety requirements during the performance of demolition works, and in particular to ensure:

1) direct supervision over those works by an employee of the H&S Office;

2) use adequate protection means indicated in relevant permission

documents;

3) training for employees which will, in particular, cover:

a) individual works distribution,

b) order of tasks to be performed,

c) occupational health and safety requirements applicable to individual activities.

Contractor is obligated to ensure that access to locations where demolition works are performed is restricted to authorized persons who have received relevant instructions.

- 6.7. Employee protection

Contractors and their Subcontractors are obligated to use individual gas detectors for minimum two gases: CO and O2:

- across the entire Primary Unit,

- in other places while high risk works are performed (especially gas hazard works, works in confined spaces).

It is forbidden to work without a detector and if a detector is out of order.

Primary Unit should be understood as the premises of the following organizational units located in:

- Dąbrowa Górnicza: Blast Furnace Plant – DG, Steel Making Plant – DG, Power & Utilities – DG,

- Kraków: Coke Plant, BF and Steel Making Plant - KR, and Power & Utilities - KR and Heat & Power Plant.

- Zdzieszowice - Coking plant

# 7. High Risk Lifting Operations

7.1. High-risk lifting operations plan covers the following activities:

- lifting operations performed with two or more cranes,

- lifting and lowering loads through holes in ceilings,

- crane operation when operator is not able to watch the load,

- operation of cranes with fixed and changeable location near overhead power lines,

- lifting ladles with liquid steel if this operation differs from  normal operation conditions,

- other lifting operations performed occasionally (e.g.: lifting requiring 
 crane overload).

7.2 Safe lifting operations plan for mobile cranes.

Definition of a mobile crane – a crane that may be equipped with a jib (jib 
tower), and may move without a special roadway, either with a load or without, with its stability is not affected by the gravitational force.

Safety requirements:

- Switching on and switching off movements stops should be forced.
 In case of a hydraulic drive with a working cylinder it is sufficient to use valves limiting working pressure to secure end position.

- Lifting height limiter – if there is a risk that the jib may collide with the gripping device when being lowered or during the telescopic extension, the lowering mechanism should be deactivated together with the jib's lifting or extending mechanism.

- Lowering limit switch should be used mostly in cases
 when it is dangerous for a crane operator, whose body position is normal, to watch the gripping device from the cabin, or when operating conditions facilitate the lowering of the gripping device below the level defined in the crane's technical specification.

- Slewing range limiters. Slewing range limiters should allow for the impact of inertial forces occurring when the crane turns, and its required stability.
In case of cranes with angle of rotation limited by operating conditions, the way a limiter is installed should make the change of the working angle of rotation impossible.

- Jib hoist limiter. In case of cranes, the working movement of the fixed jib should be controlled from a control station where it is possible to see the fixed jib in all its possible positions.

- Load limiting device. It is required to prove stability by equipping cranes with load limiting devices that would switch off all working movements when admissible load is exceeded. Load limiting device should activate when the load is exceeded by no more than 15%.

- Jib angle indicator and lifting capacity indicator. Crane in case of which the lifting capacity changes together with a change of the radius should be equipped with a lifting capacity indicator corresponding to a given radius. Cranes lifting capacity indicator should be visible from 
 the work station from which the crane is controlled. Jib angle indicator to vertical surface could also be used to identify the admissible lifting capacity. In such a case there should be a plate or a graph showing the admissible lifting capacity for a specific jib angle and radius in the operator's cabin.

- Cranes in case of which there is a risk of uncontrolled swing to the back should be equipped with a bumping block or other devices protecting against uncontrolled swing of the jib.

- Cranes threatened by wind pressure should be equipped with an
anemometer which automatically activates a sound signal.

- The hook should be equipped with devices protecting against disengagement of gripping devices or rope slings from the hook's mouth
.

Determining stability during uncontrolled swing to the back.

Correct balancing of the crane should be based on the following weight distribution when the device is operated in the following conditions:

- it is placed on a stable horizontal foundation (inclination up to 1%);

- it is equipped with the shortest possible jib determined by the works specificity, at the highest angle recommended  for this jib length;

- it is equipped with a hook, hook assembly or another gripping device lying on the ground;

- with supports without contact with ground  - calculations for crane on wheels (tires);

- it is equipped with the longest jib indicated in the specification or a jib with w
an extension, positioned at the highest recommended angle for this system, exposed to wind from the least favorable direction.

The weight distribution criteria should be fulfilled for each case of crane balancing when it turns to a position that is least stable admissible by the producer.

  - Crawler cranes.

Total load on the least loaded tip-over, side or the end edge should not be less than 15% of the total gravitational force on the crane. If the crawler crane is equipped with a caterpillar tensioner, the producer should be equipped with a warning device which is visible to the operator and informs him that the aforementioned condition is kept in case of caterpillars.

  - Cranes on wheels.

If the longitudinal axis of the crane's rotary part is turned by 90°against the longitudinal axis of the undercarriage, the total load of wheels (tires) or supports at the side of the jib should not be less than 15% of the total gravity force on the crane.

If the longitudinal axis of the crane's rotary part is parallel to the longitudinal axis of the undercarriage, total load on wheels (tires) or supports on the least loaded side of the undercarriage under the jib should not be less than 15° of the total gravitational force on the crane during operation on a different kind of ground.

Moreover restrictions related to the operation of cranes on wheels (tires) should be a warning and should be placed somewhere clearly visible for the operator. This information should specify operating conditions which require the use of supports ensuring stability in case of a tip-over.

Stability of an idle crane in case of wind.

Producer's recommendations referring to special precautions must be strictly followed if a crane is idle, as well as restrictions in crane's operation applying when it is windy.

Stability for operational load.

Stability for operational load is calculated for a crane in a least favorable 
position. Moreover all fixed loads, counterweights, accessories etc. which affect stability, should be taken into account in least favorable conditions concerning both their size and position.

Examples of edges in relation to which particular types of cranes may tip over.

Individual construction solutions should be taken into account in practice.

Cranes on wheels with blocked spring suspension

Cranes on wheels (tires)

Cranes with supports

Crawler cranes

Before High Risk Lifting Operations start, it is absolutely obligatory to prepare the High Risk Lifting Operations Plan effective under AMP procedure (Appendix no. 4).

ATTENTION!

Each person holding licenses issued by the Technical Supervision Office or Transportation Technical Supervision Office in scope of operation/maintenance of crane equipment should hold an authorization issued by ArcelorMittal Poland S.A. (Appendix no. 5)

Equipment operator is forbidden to leave a control station when a load is suspended.

# 8. Fire-related aspects and   high-risk plant

8.1. Principles of performing fire hazardous works

Works involving fire hazard, not covered by the process manual or performed outside the location used for this purpose on a permanent basis, such as works involving open fire, conducted in the fire, explosion and gas hazard areas in facilities, and in the adjacent areas and storing yards, and all repair and construction works may only be performed after a fire risk is assessed and a work site is prepared as defined in the Fire Safety Procedure for specific Units  or buildings of ArcelorMittal Poland S.A.

Assessment of fire hazard conducted by an employee of a Plant or facility will show whether a second assessment of protection measures applied for  fire hazardous works conducted by a Commission is required.

Principles of organizing fire hazardous works are defined in effective Fire safety procedures.

A permit should always be obtained from AMP supervisor before works start.

The works should be performed in a manner preventing outbreak of fire or explosion.

 Hazards created by these works must be taken into account in HIRA (Appendix no. 3).

WARNING: Fire hazard works may only be started after a fire risk assessment is performed and a work site has been prepared as defined in the effective Fire Safety Procedure.

- 8.2. Fire and explosion protection

In the facilities and adjacent areas of ArcelorMittal Poland S.A. it is forbidden to take actions which may start fire or lead to fire spreading or hinder rescue actions or evacuation.

It is forbidden to use open fire, smoke tobacco and use other objects that may result in ignition of the following materials:

- in the explosion hazard area, except for the equipment intended for such a purpose,

- in the locations where materials involving fire hazard are present,

- in the locations with other flammables, specified by ArcelorMittal Poland SA and marked in line with Polish Standards for safety marking,

- in the locations equipped with fixed extinguishing installations (to prevent them from accidental activation).

It is forbidden to hinder access to:

- extinguishers and extinguishing equipment,

- explosion venting equipment,

- water sources for firefighting purposes;

- equipment starting up extinguishing installations and controlling such installations, as well as other installations influencing fire safety of a facility,

- emergency exits,

- switches and switchboards of the electric current and main valves of gas installations

- fire-fighting equipment making it possible to conduct effective extinguishing actions in case of fire and stopping it from spreading.

- 8.3. Firefighting equipment

In the event of fire or fire risk, the available portable firefighting equipment should be used, e.g. fire extinguishers, hoses and water jets, internal hydrants, etc.

Use of any portable firefighting equipment available must be immediately reported to the works manager who provides functional extinguishing equipment.

WARNING: Fire extinguishing equipment located in the specified locations should be used exclusively for fire extinguishing. Its fixed location guarantees that it will always be there when urgently needed.

- 8.4. Fire

8.4.1 ALARM

Each person to spot fire or obtain information about fire is obligated to remain calm and not to incite panic, as well as to immediately report it to:

- persons in the vicinity of fire, exposed to its consequences;

- supervisor or owner being the main user of the facility or building,

- rescue teams (in accordance with the alarm instructions for a given location of ArcelorMittal Poland S.A.)

Fire department should be called from the nearest phone.

After the caller gets through to the Fire Department, the information to be given is as follows:

- fire location – exact address, building number and name;

- what is on fire – e.g. office room, warehouse,

- is there a threat for human life,

- are there flammable or explosive materials etc. in the area on fire or in its close vicinity;

- the phone number from which fire is being reported and full name.

WARNING: After the confirmation of obtaining the report by an officer on duty, the person reporting fire should hang up and wait for a possible verification.

8.4.2 RESCUE ACTIONS

Rescue operations should be started parallel to reporting an alarm to the rescue team. 
First of all, people in danger should be saved and then fire extinguishing actions should be started with use of portable extinguishing equipment in the building/bay, unless it threatens health and life of the person taking rescue actions.

In the event of evacuation, one should remain calm and conform to instructions given by evacuation leader and remain at the meeting point.

WARNING: Effective notification of the Fire Department is a basic activity to be taken while organizing rescue operations and is an obligation of each and every person staying within ArcelorMittal Poland S.A. premises. Immediate start of a rescue operation guarantees people safety and minimum losses due to fire.

- 8.5. Safe work in a plant at a high risk of a major industrial accident

8.5.1 WHAT AN EMPLOYEE SHOULD KNOW:

Legal regulations related to prevention of major industrial accidents

Industrial accidents are usually related to fire, explosions and releasing hazardous substances to the environment. The substances in the form of gases, vapors or liquids may pose a serious threat to people and environment.

In order to minimize threats resulting from operation of the plants which use hazardous materials, Environmental Protection Law defines requirements for this group of plants.

- Depending on the type and quantity of hazardous substance, plants are categorized into a group of plants at an increased risk of a major industrial accident or a group of plants at a high risk of a major industrial accident.

- Both groups of plants are obligated to draft an accident prevention program to be approved by a competent chief of the National Fire Department – the district Fire Department Chief in case of an increased risk, and the provincial Fire Department Chief in case of a major risk; this program is supposed to provide evidence that a plant management system guarantees protection of people and environment.

- Plants at high risk are also obligated to develop a safety report for approval by a provincial Fire Department Chief, and they are also required to prepare an internal operational and rescue plan.

Safety principles

In care of your own and your co-employees’ safety you should:

- find out whether or not there is an industrial installation with a hazardous substance close to your work station,

- read a procedure on how to proceed in the event of an industrial accident; said procedure comprising the following information:

  - types of possible risks in the so-called high-risk plants,

  - ways of communication and alerting employees in the event of the aforesaid hazards,

  - evacuation methods,

  - list of phone numbers to services responsible for undertaking operational and rescue actions,

  - other information important to your safety.

  - in order to obtain further information, please contact Fire Protection and Gas Emergency Services.

Fire Protection Service, Gas Emergency Service and Quality and Product 
Office have a database of information about hazards, which is included in the Plant Registration, Accident Prevention Program, Safety Report, Internal Operational and Rescue Plan and is maintained on an ongoing basis.

8.5.2 UNIT IN DĄBROWA GÓRNICZA

qualified as a plant at high risk of a major industrial accident

In accordance with Environmental Protection Law and respective regulations, ArcelorMittal Poland S.A. provided official report to the Provincial Unit of the National Fire Department and Provincial Inspector of Environment Protection on being a plant at high risk of a major industrial accident. In Dąbrowa Górnicza Unit of ArcelorMittal Poland S.A. there are two gas generation installations involving hazardous substances listed below, as a result of which the plant is considered a plant at high risk of a major industrial event:

- Blast furnace gas

Blast furnace gas is generated in a blast furnace process.

Main elements of BF gas installations and distribution network are as follows:

- Blast furnace No. 2

- BF2 hot stoves

- Blast Furnace no. 3

- BF3 hot stoves

- BF2 gas cleaning station

- BF3 gas cleaning station

- Gas flares

- BF pipelines between different departments

- BF gas distribution network

- BOF-gas

BOF gas is generated in a steel-making process.

Main elements of BOF gas installations and distribution network are as follows:

- BOF-1

- BOF-3

- BOF1 gas cleaning installation

- BOF3 gas cleaning installation

- Gas flares

- Gas holder

- BOF gas pipelines between different departments

- Gas mixing and reduction stations

- BOF gas distribution network

- Mixed gas network

8.5.3 UNIT IN KRAKÓW

qualified as a plant at high risk of a major industrial accident

In accordance with Environmental Protection Law and respective regulations, ArcelorMittal Poland S.A. provided official report to the Provincial Unit of the National Fire Department and Provincial Inspector of Environment Protection on being a plant of at high risk of a major industrial accident. In Kraków Unit of ArcelorMittal Poland S.A. there are the following hazardous substances, as a result of which the plant is considered a plant of at high risk of a major industrial accident:

- Blast furnace gas

Blast furnace gas is generated in a blast furnace process.

Main elements of BF gas installations and distribution network are as follows:

- Blast furnace No. 3,

- BF3 hot stoves

- Blast furnace No. 5,

- BF 5 hot stoves

- Gas cleaning plant No. 2.

- Gas cleaning plant No. 3.

- Gas burners

- Power Plant's boiler house

- Steam superheating station

- Wagons defrosting stations.

- Gas stations.

- BF gas pipelines between different departments.

- BF gas transmission network.

- Coke-oven gas

Coke-oven gas is generated in coke-oven batteries of the Coking Plant.

Major coke oven gas receivers are as follows:

- Blast furnace No. 3,

- BF3 hot stoves

- Blast furnace No. 5,

- BF5 hot stoves

- Gas cleaning plant No. 2.

- Gas cleaning plant No. 3.

- Gas burners

- Power Plant's boiler house

- Steam superheating station

- Wagons defrosting stations.

- Gas stations.

- BF gas pipelines between different departments.

- COG pipelines between different departments.

- Coke Ovens

- Exhaust fans.

- Ammonia Plant.

- Benzol Plant.

- Desulfurization plant.

- Valve station.

- COG distribution network.

- Coke tar

Occurs in Coking Plant

- Coke-oven benzol and stripping oil,

Occurs in Coking Plant

- Oxygen

Oxygen for the needs of production plants in Kraków Unit is generated by the following installations:

- Oxygen Generating Plant Air Liquide – Polska Sp. z o.o.

- Oxygen Unit No. 4

The basic facilities of oxygen production installations are as follows:

- Oxygen Block No. 4

- Air Compression Plant  K-1500

- Oxygen Block KAr  30

- Pumping Station No 32

- Pumping Station No 33

- Water Treatment Plant

- Oxygen transmission network

- Anhydrous ammonia

Used in Cold Rolling Mill

Major elements of anhydrous ammonia receivers:

- Anhydrous ammonia storage facility.

- Protective Gas Generation Plant 1.

- Protective Gas Generation Plant 2.

- Hot Dip Galvanizing Line.

- Magnesium

Present in the Steel Making Plant and the HM Desulfurization Station.

- Sulfur

Present in the Coking Plant and the Power Plant.

- Coal management.

Present in the Power Plant and the BF Plant.

- Petroleum by-products

Present on the premises of the entire Unit.

K. Natural gas

Present on the premises of the entire Unit.

8.5.4. UNIT IN ZDZIESZOWICE

In ZKZ Coke Oven gas is treated in three mutually dependent processes. Coke Oven gas from batteries 3-6 is treated in a low pressure system (pressure of ca. 0.2 bar), where mainly tars and ammonia are removed; gas from batteries 7-12 is also treated in the low pressure system, where tars, hydrogen sulfide, ammonia, benzol and naphthalene are removed. Total volume of gas treated in those two process installations is ca. 200,000.0 Nm3/h. The aforementioned stream of gas is then routed into two directions:

-for firing the heating systems of coke oven batteries 3-12 (ca. 50%)

-for compression and deep gas treatment (the remaining 50%, i.e. ca. 100,000.0 Nm3/h)

Due to requirements of external clients (external sales and an internal heat and power plant) deep treated gas meets the following purity standards (according to an old no longer valid PN-87/C-96001 standard for gaseous fuels……):

- benzol content (BTX fraction): max. 3.0 g/Nm3

- naphthalene content:               max. 0.10 g/Nm3 (in summer)

max. 0.05 g/Nm3 (in winter)

- hydrogen sulfide content:        max. 0.02 g/Nm3

- ammonia content:                 	  max. 0.005 g/Nm3

In the aforementioned high pressure process installation the coke oven gas is fed to two sets of three-stage piston gas compressors (total of 10, max. capacity of 19,500.0 Nm3/h each), where it is compressed to max. pressure of 12.5 bar, and water and benzol condensate is partially removed. Subsequently, after cooling in 1st stage horizontal tube coolers to 25-30 C the gas is fed to two pressure absorption systems working in parallel.

8.5.4. Alerting and evacuation rules to be followed in the event of a major accident

Principles for raising the alarm

Employees will be alarmed of a risk through company sirens. An alarm can also be raised by the Fire Protection Service and Gas Rescue Services (e.g. if sirens do not function) using a fire engine fitted with a loud speaking system.

Alarm signals

| ALARM TYPE | HOW TO RAISE AN ALARM |  | HOW TO CANCEL AN ALARM |  |
| --- | --- | --- | --- | --- |
|  | USING SIRENS | USING MASS MEDIA | USING SIRENS | USING MASS MEDIA |
| ALARM OF NATURAL DISASTARS AND ENVIRONMENTAL RISK | MODULATED BEEPING SIGNAL | ORAL ANNOUNCEMENT ABOUT A RISK AND A PROCEDURE TO BE FOLLOWED BY AREA RESIDENTS REPEATED THREE TIMES | CONTINUOUS 3-MINUTE SOUND | ORAL ANNOUNCEMENT REPEATED THREE TIMES: ATTENTION! ATTENTION! I CANCEL A DISASTER ALARM FOR ………………….. |

Evacuation principles and conditions

Once an alarm is raised each and every employee of ArcelorMittal Poland S.A. and each and every Contractor's employee should immediately go to the evacuation site indicated in the operational and rescue plan, or follow the orders of the evacuation supervisor. Excerpts of the evacuation plan with maps showing evacuation points are available at Plants.

WARNING: Employees directly involved in elimination of industrial accident consequences are not subject to evacuation. They should immediately proceed to Gas Supervision and Emergency Station in order to equip themselves with respiratory protection equipment or use equipment located in emergency cabinets.

Principles of evacuation management

A foreman, department manager, shift leader in relation to their subordinate employees are in charge of evacuation in the relevant organizational unit of ArcelorMittal Poland S.A. Their tasks include:

- Quick and efficient evacuation to the place indicated in the "plan". Choice of the meeting point depends on a direction in which toxic cloud spreads and accident location.

- Checking the number of evacuated employees.

- Communicating information on the number of evacuated employees and the number of employees who stayed in the hazard area to a coordinator of rescue activities.

- Continuous cooperation with a rescue action coordinator on changing weather conditions (direction of spreading of a toxic cloud) and industrial event scale.

WARNING:

Evacuation operation should be performed against the direction of the wind, and upwind from a contaminated zone.

8.5.5. Principles of how to proceed after the alarm was raised

In the event of an industrial accident with hazardous substances being involved, follow the procedure presented below:

| 1. After the alarm is raised: | 2. Inform your colleagues |
| --- | --- |
| 3. Abandon the area at risk and proceed to the indicated evacuation point. Excerpts of the evacuation plan with maps showing evacuation points are available at Plants. |  |
| 4. Follow orders of a person in charge of evacuation | 5. Communicate the necessary information about the number of evacuated employees and employees who remained in the hazard zone to a person in charge of evacuation. |
| 6. Do not leave the meeting point until the alarm is canceled. | E |

# 9. Accidents and first aid

- 9.1. Accidents at work

Each and every accident or an incident which caused or might have caused an injury must be immediately reported to:

- H&S Supervisor representing the Contractor (according to the Agreement);

- H&S Coordinator or representative of ArcelorMittal Poland SA (e.g. Repair Manager, Investment Manager, etc.),

- H&S Office of AMP

The Contractor shall be obligated to send a filled in "H&S Alert - Preliminary information about the accident” form (template in Appendix No. 7), to H&S Office within 12 hours of the incident.

The circumstances and reasons for accidents should be determined in cooperation with a representative of the H&S Office of ArcelorMittal Poland S.A.

No later than within 7 days of the end of the post-accident investigation but no later than within 21 days of the accident at work or the aforementioned incident a copy of a post-accident report or an incident certified to be true by the Contractor shall be submitted to H&S Office. If the incident has been reported to the National Labor Inspection, H&S Office shall be immediately informed thereof. A copy of the National Labor Inspection's audit report with appendices certified to be true by the Contractor shall be submitted to H&S Office within 7 days of its signing.

The Contractor is obligated to ensure participation of employees involved in an accident in a team appointed by ArcelorMittal Poland S.A. to analyze circumstances and causes of the incident, to present preventive actions, people responsible for their implementation, deadlines for completion and to inform H&S Office about their completion.

Each Contractor is obligated to submit statistical information in scope of work safety by the fifth working day of the following month for the previous month or directly after works completion to the organizational unit of H&S Office (on the day of signing the handover protocol at the latest) relevant for particular Company location where they conduct works. Previous month’s statistics should include:

- number of man-hours worked on the premises of ArcelorMittal Poland S.A. (Contractor's and Subcontractor's);

- total number of accidents at work (Contactor's and Subcontractor’s employees) on the premises of ArcelorMittal Poland S.A. including:

- number of Lost Time Injuries

- days of sick leave lost due to accidents at work in a given month.

Reporting accident at work does not release the Contractor from an obligation to conduct an accident investigation as defined in the labor law.

- 9.2. Procedure in the event of an accident at work

Contractor's obligations:

- immediately provide first aid to a person who suffered an accident

- call rescue services

- secure the accident site against access of unauthorized persons and unnecessary start of machines and other process equipment stopped due to the accident. It is forbidden to move machines, equipment and other items which caused the accident. Changes are acceptable if necessary to save people or assets, or to prevent danger.

WARNING: Delays in calling the rescue services (an ambulance, fire brigade, gas rescue) for instance to inform the contractor's employees first are FORBIDDEN. Each and every such situation shall be treated as a major breach of rules effective in ArcelorMittal Poland S.A.

- 9.3. Providing first aid

Each Contractor's employees working on the premises of ArcelorMittal Poland S.A. must include people trained on the provision of first aid, and - respective for the risks - they must be provided with relevant first aid kits. The first aid kits must available in locations where work is performed.

Each person that suffered from an accident must receive first aid.

Use of the first aid kit by trained personnel is not restricted – the fact of its use must be recorded in the first aid kit records maintained by the Contractor and reported to ArcelorMittal Poland S.A. representative.

First aid – each first aid kit contains a list of persons trained in scope of providing first aid.

Medical aid (calling an ambulance) – execute in accordance with instructions and emergency call board effective for all ArcelorMittal Poland S.A locations.

Emergency telephone numbers are shown on the emergency calls board in each ArcelorMittal Poland unit.

Emergency telephone numbers are shown on the emergency calls board in each ArcelorMittal Poland unit.

- 9.4. Near misses

After a dangerous near miss occurs, a witness of such an incident is obligated to immediately report it to their superior, who reports it to OH&S Coordinator and H&S Office (report template in Appendix No. 8)

Each near miss must be analyzed by the Contractor.

# 10. Use of machines, equipment and tools owned by ArcelorMittal Poland S.A.

Contractors may use machines, tools and equipment owned by ArcelorMittal Poland S.A. based on a written permission issued by ArcelorMittal Poland S.A. The consent for the operation of the machines, tools and equipment owned by ArcelorMittal Poland S.A. shall be issued on behalf of the ArcelorMittal Poland S.A. by the responsible/managing person. The permit for machines operation is issued to individuals after relevant qualifications have been certified and after the training conducted by the subcontractor has been completed. The responsibility for operating tools, machines and equipment used lies with the Contractor.

# 11. Environmental protection

In relation to the activities conducted, the Subcontractors are obligated to:

- hold all the required permits for use of environment in their scope of activity,

- use best available techniques related to environmental protection,

- manage waste materials generated in the process of their activities on premises of ArcelorMittal Poland S.A., as per the obtained permits and provisions of the Contract for Work Performance on Premises of ArcelorMittal Poland S.A., and effective legal regulations,

- observe rules on waste segregation and proper storage,

- provide ArcelorMittal Poland S.A. with all the information on all environmental aspects related to the activity conducted,

- minimize the environmental impact of the activity conducted.

Discharge of chemicals to the sewage system is strictly prohibited!

# 12. Inspections conducted in the place of work

Employees of H&S Office as well as each employee of ArcelorMittal Poland S.A. responsible for the control of progress in works performed by the Contractor shall be authorized to conduct safety audits in the place of work performance and to prepare reports.

If requested by an AMP representative, the Contractors are obligated to provide documents concerning the scope of the audit.

Any and all observations recorded during such audits by the representative of ArcelorMittal Poland S.A., which would suggest the existence of a potential hazard, may result in the discontinuation of works and the continuation thereof after implementing necessary measures and corrective actions agreed with and approved by representatives of ArcelorMittal Poland S.A.

Contractors and their Subcontractors are subject to continuous H&S assessment on the premises of ArcelorMittal Poland S.A. The observed violations are reported in HandS system – the Contractor is informed by an email (sent to the address specified by them) and is obligated to take action immediately to eliminate the violation, as well as ensure that such event shall not reoccur. The Contractor shall be obliged to inform a person indicated in the email that is a person in charge of the area or the work on the part of ArcelorMittal Poland S.A. about the aforesaid actions.

The results of these audits will be taken into account during the meetings with Contractors that seek to obtain new orders.

# 13. Contractual penalties

In case of breach of H&S regulations the Contractor shall pay ArcelorMittal Poland S.A. contractual penalties as per Appendix No. 9 - Catalogue of fines which defines categories of violations of H&S regulations and amounts of fines for a single violation. The fines are cumulative.

The Contractor shall be charged with a fine based on a note issued by ArcelorMittal Poland S.A. Failure to pay the contractual penalty shall constitute a basis for ArcelorMittal Poland S.A. to terminate the breached Contract for Work Performance on Premises of ArcelorMittal Poland S.A. without notice. In such case it is assumed that termination of the Contract for the Performance of Work on the Premises of ArcelorMittal Poland S.A., in relation to which violation occurred, was due to the fault of the Contractor.

Debit notes will be issued based on violations identified by employees of ArcelorMittal Poland S.A. and registered in HandS.

The payment of all or any of the contractual penalties does not prejudice the possibility of claiming supplementary compensation based on general principles (in the amount exceeding the reserved contractual penalties).

14.	Activities aimed at remedying violations.

- The Contractor acknowledges that it is ArcelorMittal Poland S.A. priority to ensure safety of Employees, employees of ArcelorMittal Poland S.A. and all other persons on the Premises of ArcelorMittal Poland S.A. and to prevent property losses and personal injuries. Should a Contractor or a Subcontractor (or Employees and all other persons for whom the Contractor or the Subcontractor is responsible) violate H&S regulations and principles, as well as other effective legal regulations which may result in a serious, fatal or severe accident by fault of the Contractor, Subcontractor, Employee or other person for whom the Contractor or Subcontractor is responsible or should the “zero alcohol” rule be violated  – if at least three Contractor’s Employees or Subcontractor’s Employees are caught attempting to enter, exit or remain on the Premises of ArcelorMittal Poland S.A. while under the influence of alcohol, drugs or other intoxicants, or if they are caught in possession of alcohol, drugs or other intoxicants while remaining on the Premises of ArcelorMittal Poland S.A.,  ArcelorMittal Poland S.A, irrespective of imposing contractual penalties or other means, ArcelorMittal Poland S.A.  will be able to apply the means defined in clause 2.

- ArcelorMittal Poland S.A. has the right to suspend co-operation with a Contractor for the period of 6 months (at its own discretion), which means that ArcelorMittal Poland S.A. will not send new requests for quotation or new offers on cooperation to the Contractor, nor will it start any negotiations on new co-operation.

- The Contractor will be notified of the decision made by ArcelorMittal Poland S.A. immediately after it has been made.

- When the period of discontinuation referred to in clause 2 is over, if the Contractor showed that they took required and effective action to prevent further violation of H&S regulations, and ArcelorMittal Poland S.A. accepted those actions, the Contractor may apply to be re-entered on the service suppliers list of ArcelorMittal Poland S.A., after completing the following procedure:

- Preparing information about actions the Contractor will undertake to perform to eliminate future H&S violations. Information shall list actions taken by the Contractor's Board of Directors (Owner) concerning:

- solution to the problem of the specific violation

- involvement of management in improvement of H&S during work performed on the premises of ArcelorMittal Poland (e.g. audits)

b) 	Presenting the prepared information at a meeting with representatives of ArcelorMittal Poland S.A.

5. Based on discussions, decision will be made to resume co-operation with Contractor and on conditions of resuming the same (understood as occupational health and safety requirements). The Contractor shall be informed about the decision in writing by Services Procurement of ArcelorMittal Poland.

# 15. Declaration

I hereby confirm that I have received a copy of the Contractor’s Safety Manual of ArcelorMittal Poland S.A.

I shall get acquainted with its provisions and I shall observe all the regulations and principles 
included herein, as well as all additional provisions and principles concerning the work performed by my company and my Contractors on premises of all ArcelorMittal Poland S.A. Units.

Safe work and PPE use are mandatory for tasks performance within the premises of ArcelorMittal Poland S.A.

In the event of a breach of H&S principles and regulations by employees reporting to me, I shall submit to disciplinary proceedings incl. exclusion of my company from 
performance of the task commissioned.

I also acknowledge that this document and regulations and principles hereof do not release me from legal liability in scope of work safety during task performance.

I declare that all my employees and employees of my Subcontractors as well as persons employed on the basis different than employment contract shall observe the provisions hereof.

| Company name |  |
| --- | --- |
| Full name (print) |  |
| ID no |  |
| Signature of Contractor/Employer* |  |
| Date |  |

*or a person acting on their behalf