TECHNICAL SPECIFICATION no 04a/AMTP/2022

concern the design/supply of the in line NDT Flux leakage system including auxiliaries.

Project:. “Manufacturing innovative products on a hybrid line for producing black and metallic coated tubes with simultaneous tightening of dimensional tolerances, using a flux leakage method for a qualitative testing”

Project no.: POIR.01.01.01-00-0510/18

in ArcelorMittal Tubular Products Kraków

SPECIFICATION INDEX

# TECHNICAL SPECIFICATIONS	Page

1.0	LINE SPECIFICATIONS	3

1.1	INTRODUCTION	3

1.2	PERFORMANCE REQUIREMENTS	8

1.3	LINE PARAMETERS	8

1.4	SCOPE OF SUPPLY	14

1.5	SPARES	15

1.6	SERVICES	15

2.0	EQUIPMENT SPECIFICATIONS	17

2.1	GENERAL	17

2.2	MECHANICAL SPECIFICATION.	18

2.3	ELECTRICAL SPECIFICATION	19

2.4	MILL CONTROL	21

2.5	LEVEL 2 COMPUTER INTERFACE REQUIREMENTS	22

3.0	SUPPLEMENTAL INSTRUCTIONS TO BIDDERS	23

3.1	DELIVERY OF EQUIPMENT	23

3.2	DOCUMENTS TO BE SUPPLIED WITH THE BID	24

3.3	DOCUMENTS TO BE SUPPLIED AFTER A CONTRACT	25

3.4	WORKS BY BUYER	29

3.5	DEADLINE FOR PROJECT IMPLEMENTATION	29

4.0	PAYMENT CONDITIONS	29

4.1	PROVISIONAL ACCEPTANCE TEST 	30

4.2	PERFORMANCE ACCEPTANCE CERTIFICATE	31

1.0	LINE SPECIFICATIONS

1.1	INTRODUCTION

The immediate goal of the project is (taking into account all the quality requirements for      the manufactured products and technical and technological consequences) the production       of innovative pipes, characterized by increased quality parameters regarding corrosion protection, as well as dimensional deviations on the nominal diameter and length, the quality of the linear weld and the accuracy of the methods testing to verify the material for longitudinal defects. An innovative technology is also the result that will have a positive impact on the environment by reducing CO2 emissions and reducing material consumption compared to the production of hot-rolled pipes.

The implementation of the project will allow to:

- manufacture and place innovative products in terms of corrosion protection on the domestic and European market;

- manufacture products with narrow tolerances in diameter and length, for construction, pressure and installation applications as well as for the transport of gas and crude oil, allowing for savings not only in the exploitation phase, but also in the production phase at the end user;

- obtaining high-quality products that are not produced in Poland;

The subject of the contract indicated in this specification concerns the project entitled “Manufacturing innovative products on a hybrid line for producing black and metallic coated tubes with simultaneous tightening of dimensional tolerances, using a flux leakage method for a qualitative testing” (contract number for co-financing the project: POIR.01.01.01-00-0510/18, co-financed under sub-measure 1.1.1 of the Intelligent Development Operational Program 2014-2020, co-financed by the European Regional Development Fund.

Due to the obligation of the Company to apply the principle of competition, this technical specification constitutes a detailed description of the subject of the contract allowing the preparation of offers by the Bidders.

This specification has been prepared with the utmost care to define a full, unambiguous and exhaustive description of the subject of the contract, so as to enable contractors to define all their obligations and risks and to responsible calculation of the price and other elements of the offer.

All purchases, services and delivery subject to this inquiry must be included and cooperate with the existing infrastructure and equipment in the Company and must meet the same technological standards. In connection with the above, the necessity to maintain the same technological conditions and the necessity to maintain the unification of devices resulting from the expansion of the existing infrastructure determined the provisions of this specification. The provisions used are justified by the need to ensure the efficient implementation of the project. The indicated provisions do not impose on the Contractors the obligation to apply the indicated solutions, but only inform about the minimum parameters and standards. The use of certain types of solutions is not obligatory but only exemplary. The indications regarding the expected technical parameters, and indications regarding specific types and producer names are general in nature, referring only to examples of indications of equivalent products and do not constitute the only accepted solution. On this basis, the contracting authority allows for equivalent solutions.

Bidders are expected to submit an offer taking into account the requirements of this Technical Specification

The offer must be complete in all respects and must include all elements / devices necessary to achieve proper construction, operation and maintenance of the installation.

The tenderer is obliged to get acquainted with this specification and make sure that the installation is technically feasible, as well as assume full responsibility for the guaranteed operation of the delivered installation and equipment in terms of efficiency, parameters and efficient and reliable operation.

The detailed scope of works being the subject of the Inquiry is presented later in this study.

Contact persons:

Jakub Dziurdzia

jakub.dziurdzia@arcelormittal.com

+48 606 815 712

Artur Fiałek

artur.fialek@arcelormittal.com

+48 668 312 518

This Specification covers the technical requirements for the design/supply – on DDP Kraków basis -  of an in line NDT Flux leakage system including auxiliaries (described in detail in item 1.4.1).

These devices will be part of the project to build a Pilot Line for the production of Ø76,1– Ø168,3mm OD tube and 60x60 - 120x120 hollow structural sections.

Tube Mill (Pilot Line) which consists of devices from below specification is a part of research and development project named: “Manufacturing innovative products on a hybrid line for producing black and metallic coated tubes with simultaneous tightening of dimensional tolerances, using a flux leakage method for a qualitative testing”. The project result is to manufacture innovative tubular products (cold formed black pipes and with metallic coatings), which allow our company introduction of innovative products in domestic and Europe market in terms of:

- Anti-corrosion protection,

- Products with tightened dimensional and length tolerances for purposes of construction, pressure application, installation and for gas and oil transport.

As the result of the project we plan to obtain high quality products which are not produced in Poland. The use of all devices in the technological line will let us conduct the production process in a continuous and very effective way minimizing energy consumption, reducing manipulation, increasing work safety, which will affect the environment and employee safety.

NDT Flux leakage system which is the subject of the following technical specifications will be the part of a complete production line consisting of the following components which are the subject of separate technical specifications:

- Tube mill,

- Non-destructive testing system for longitudinal welding,

- Non-destructive testing system for the full body (by the flux leakage method),

- Internal seam removal system,

- High frequency welding machine,

- Chamfering machine

- Automatic bundling machine,

- Marking devices,

- Pipe diameter measuring system.

The scope of supply includes but is not limited to:

- Design and preparation of the technical documentation  (mechanical and electrical documentation) of the in line NDT Flux leakage system including auxiliaries in accordance with Equipment Specification request  described in detail in point 2.0 of this document.

- Supply of individual elements of the in line NDT Flux leakage system including auxiliaries in accordance with Scope of Supply request described in detail in item 1.4 of this document.

- Deliveries of individual elements of the in line NDT Flux leakage system including auxiliaries in accordance with Delivery of Equipment request described in detail in item 3.1 of this document

- Assembly, installation in turnkey mode, commissioning and start-up of individual components of the in line NDT Flux leakage system including auxiliaries in accordance with Turn-key Installation request described in detail in item 3.1 of this document.

- Integration with automation and structure of the Tube mill & accessories described in item 1.1  Introduction of present document.

- Conducting training in accordance with  Training request described in detail in item 1.6.2. of present document.

1.1.1.	Specification contents.

This Specification contains information on the natural environment, the location of the Ordering Party, the required certificates and technical standards, the scope of the Bidder’s work, the Buyer's rights, requirements for the Bidder’s technical capabilities, preliminary work schedule, requirements for availability, interchangeability, quality and safety, and other information required in the Offer.

1.1.2. Quality, units, standards and regulations.

All technical requirements must comply with environmental standards, as well as with standards for electrical and energy installations. In addition, they must comply with engineering standards such as DIN, GOST, BS, and PN (or equivalent).

National and international standards

All equipment and installations at ArcelorMittal Tubular Products Kraków must be covered by the warranty and made in accordance with applicable legal regulations (and, if applicable, according to the scope of work / deliveries):

-  Machinery Directive 2006/42/WE,

-  Pressure Equipment Directive 2014/68/WE

-  Low Voltage Directive 2006/95/WE,

- Fabricated / welded structures - DIN 8570 BF,

- Machining - ISO 2768 / IS 2102,

- Polish construction law,

- Standards required to obtain CE certificate.

- Directives, standards and legal regulations concerning pressure components subject to the requirements of the technical supervision of the Office of Technical Inspection (UDT) and the Central Laboratory of Technical Inspection (CLDT). (Concerns the operational safety of devices, including explosion dampers, exhaust valves, safety valves and pressure pipelines subject to UDT supervision and protection automatics subject to CLDT supervision).

- Directives, standards and legal acts concerning fire protection systems and installations for devices, installations and buildings.

International technical standards:

CEN European Committee for Standardization;

CENELEC European Committee for Electrotechnical Standardization;

DIN Deutsche Industrie Normen (German Institute for Standardization);

EN European Standard (European standard);

ETSI European Telecommunications Standards Institute;

ISO International Organization for Standardization.

In case of standards other than those listed above, their European (EN) or Polish (PN) equivalents should apply.

In addition, the Bidder is obliged to familiarize themselves with the standards of ArcelorMittal Tubular Products Kraków regarding the performance of works and with the provisions on health and safety at work, and is obliged to comply with them consistently.

1.1.3. Documentation standards

File formats - Company standard:

1. Documents: * .doc, * .pdf, * .xls (Microsoft Word 2003, Adobe Reader, Microsoft Excel 2003);

2. Construction and mechanical documentation: * .dwg, * .dwf (AutoCAD ver. 2000 or higher,

3. Photos and pictures: * .jpeg;

1.1.4. Safety book

During the performance of all stages of work (Investment) on the premises of ArcelorMittal Tubular Products Kraków, the safety requirements contained in the Safety Book, including all attachments, including Fatal Accident Prevention Standards must be complied with and applied on an ongoing basis.

- ST 000 Health and Safety Policy

- ST 001 Insulation

- ST 002 Confined spaces

- ST 003 Work at height

- ST 004 Railway safety

- ST 005 Audits

- ST 006 Vehicles and driving

- ST 007 Lifting devices and lifting operations

- ST 008 Contractors

- ST 009 Alarming

- ST 010 Safety indicators

- ST 011 Incident Investigation

- ST 012 Work in gas-hazardous areas

- ST 014 Hazard Identification and Risk Assessment - HIRA

- ST 015 Golden Rules

- ST 018 Securing of loads

- ST 201 Health and safety specification

- ST 301 Mobile phones

ATTENTION:

In case of different requirements in subsequent norms or standards compliant with the above-mentioned ones, the more demanding (more restrictive) norms or standards should be applied!

1.2	PERFORMANCE REQUIREMENTS

The performance requirements by the Bidder under the contract, apart from warranty provided for in the contract, has to pertain to the performance of the equipment specifically described in this specification with regard to:

a.	Productivity of the equipment

b.	Quality of products from the equipment

c.	Reliability of the equipment

The Bidder has to specifically ensure that the equipment together with the Buyer's supplies has to have the productivity stipulated in this specification and that the products of the equipment has to be in accordance with the quality specified in this specification.

1.3	LINE PARAMETERS

1.3.1	Specification of products manufactured on the pilot line for the production of pipes.

The following table contains the specification of the material to be produced on the Tube Mill and tested on the NDT Flux leakage system that is the subject of specification. It specifies the minimum technical parameters and tolerances of the material. All components and devices has to be adapted to process the raw material defined in this way.

| Tube O.D. | min. | 76,1 mm. |
| --- | --- | --- |
| Tube O.D. | max. | 168,3 mm. |
| Tube length | (min. – max.) | 5.5 – 12.8m |
| Straightness | max. | 1mm per meter |
| Tube wall thickness | (min. – max.) | 2.0 – 6.0 mm |
| OD tolerance | (min. – max.) | ± 0.6% or ± 0.3 mm whichever is greater |
| Ovality tolerance | max. | 1% of the outside diameter |
| Mill speed | (min. – max.) | Max 75 m/min |
| Operating temperature | Celsius | -15 : +45 |
| Compressed air | bar | 6 |
| STANDARD | EN 10219-1 and EN 10219-2 ASTM A 53 Grade A and B EN 10255, EN 10217-1,-2 EN 10305-3, EN 10224, EN ISO 3183, EN 10296-1, |  |

Production speed (meters per minute) as minimum required to process   incoming material from the tube mill is presented in the table below:

| WT | 2,0 | 2,3 | 2,6 | 2,9 | 3,2 | 3,6 | 4,0 | 4,5 | 5,0 | 5,6 | 6,0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Diam | Mill Speed [m/min] |  |  |  |  |  |  |  |  |  |  |
| 76,1 | 80 | - | - | 75 | - | - | 55 | - | 50 | - | 50 |
| 82,5 | - | - | - | - | - | - | - | - | - | - | - |
| 88,9 | - | - | - | - | - | - | - | - | - | - | - |
| 95 | - | - | - | - | - | - | - | - | - | - | - |
| 101,6 | - | - | - | - | - | - | - | - | - | - | - |
| 114,3 | - | - | - | 55 | - | - | 50 | - | 50 | - | 40 |
| 127 | - | - | - | - | - | - | - | - | - | - | - |
| 139,7 | - | - | - | 50 | - | - | 40 | - | - | - | - |
| 152,4 | - | - | - | - | - | - | - | - | - | - | - |
| 159 | - | - | - | - | - | - | - | - | - | - | - |
| 168,3 | - | - | - | - | - | - | 40 | - | - | - | 30 |

| WT | 2,0 | 2,3 | 2,6 | 2,9 | 3,2 | 3,6 | 4,0 | 4,5 | 5,0 | 5,6 | 6,0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Diam | Mill Speed [m/min] |  |  |  |  |  |  |  |  |  |  |
| 60x60 / 80x40 | 75 | - | - | - | 70 | - | 50 | - | 45 | - | 45 |
| 80x50 | - | - | - | - | - | - | - | - | - | - | - |
| 70x70 / 90x50 / 100x40 | - | - | - | - | - | - | - | - | - | - | - |
| 100x50 | - | - | - | - | - | - | - | - | - | - | - |
| 80x80 / 100x60 / 90x70 | 55 | - | - | - | 50 | - | 50 | - | 45 | - | 35 |
| 90x90 / 140x40 / 120x60 | - | - | - | - | - | - | - | - | - | - | - |
| 100x100 / 120x80 / 150x50 | - | - | - | - | 50 | - | 50 | - | 35 | - | 30 |
| 110x110 / 140x80 | - | - | - | - | - | - | - | - | - | - | - |
| 120x120 / 160x80 / | - | - | - | - | - | - | - | - | - | - | - |
| 127x127 | - | - | - | - | - | - | - | - | - | - | - |
| 180x80 | - | - | - | - | - | - | 35 | - | 30 | - | 25 |

1.3.2	Product Specifications

General:	All tubing produced has to conform EN 10217-1, EN 10217-2 and EN ISO 3183 specifications as a minimum unless otherwise indicated in this specification.

Surface Condition: 	Equipment has to be designed and supplied to keep the finished product free of marks and scratches.

1.3.3	Production mix

Together with the offer, the Bidder will provide a bottleneck analysis based on the production line speed table from point 1.3.1 showing cycle times split into individual production range dimensions (pipe diameter x wall thickness) in accordance with the table below for given production processes.

For each item in the table (below) the equipment capacity has to be given, taking into account all manufacturing processes. This means that the values given below will constitute a declaration of the possibility of their fulfillment by the Bidder. Lack of declaration of compliance with the requirements set out in the table below will result in the rejection of the offer.

Below provided speeds concern minimum required by this specification which will be a subject for Provisional Acceptance Test.

| WT | 2,0 | 2,3 | 2,6 | 2,9 | 3,2 | 3,6 | 4,0 | 4,5 | 5,0 | 5,6 | 6,0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Diam | Mill Speed [m/min] |  |  |  |  |  |  |  |  |  |  |
| 76,1 | 80 | - | - | 75 | - | - | 55 | - | 50 | - | 50 |
| 82,5 | - | - | - | - | - | - | - | - | - | - | - |
| 88,9 | - | - | - | - | - | - | - | - | - | - | - |
| 95 | - | - | - | - | - | - | - | - | - | - | - |
| 101,6 | - | - | - | - | - | - | - | - | - | - | - |
| 114,3 | - | - | - | 55 | - | - | 50 | - | 50 | - | 40 |
| 127 | - | - | - | - | - | - | - | - | - | - | - |
| 139,7 | - | - | - | 50 | - | - | 40 | - | - | - | - |
| 152,4 | - | - | - | - | - | - | - | - | - | - | - |
| 159 | - | - | - | - | - | - | - | - | - | - | - |
| 168,3 | - | - | - | - | - | - | 40 | - | - | - | 30 |

The following table shows the requirements for the wall thickness range for the pipe outer diameters tested on NDT Flux leakage system. The following table presents the minimum requirements for the scope of the production mix (pipe diameter and wall thickness) of the offered equipment. Lack of declaration of compliance with the requirements set out in the table below will result in the rejection of the offer.

| WT | 2,0 | 2,3 | 2,6 | 2,9 | 3,2 | 3,6 | 4,0 | 4,5 | 5,0 | 5,6 | 6,0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Diam | Steel grades |  |  |  |  |  |  |  |  |  |  |
| 76,1 |  |  |  |  |  |  |  |  |  |  |  |
| 82,5 |  |  |  |  |  |  |  |  |  |  |  |
| 88,9 |  |  |  |  |  |  |  |  |  |  |  |
| 95 |  |  |  |  |  |  |  |  |  |  |  |
| 101,6 |  |  |  |  |  |  |  |  |  |  |  |
| 114,3 |  |  |  |  |  |  |  |  |  |  |  |
| 127 |  |  |  |  |  |  |  |  |  |  |  |
| 139,7 |  |  |  |  |  |  |  |  |  |  |  |
| 152,4 |  |  |  |  |  |  |  |  |  |  |  |
| 159 |  |  |  |  |  |  |  |  |  |  |  |
| 168,3 |  |  |  |  |  |  |  |  |  |  |  |
|  | Max grade S355/L355/P355/E355 |  |  |  |  |  |  |  |  |  |  |

The potential supplier should include in the offer above table to confirm that the Flux Leakage testing system is able to detect F2 testing level acc. to EN ISO 10893-3 and carrying out leak test acc. to EN ISO 10893-1, on the presented OD/WT range using max. grade S355/L355/P355/E355 (with chemical composition) acc. to the standard for production of carbon steel material (ferromagnetic).

1.3.4	Design parameters of NDT Flux leakage system.

When designing the equipment the Bidder must take into account the following parameters and conditions described below. It specifies minimum technical parameters and tolerances for finished products (steel pipes). All components and devices must be adapted to produce the range of finished products defined in this way. Lack of declaration of compliance with the requirements set out in the table below will result in the rejection of the offer.

It is the Buyer’s intent to be able to have an equipment changeover time (last good tube to first good tube) of 20 minutes maximum for full size changing.

The Bidder has to include in the offer description of the changeover and the time (last good tube to first good tube) for full size changing.

Bidder has to include all required equipment, tools, etc. required to achieve this in the base bid.

The equipment has to be designed to minimize the manpower required to fulfil the tubing requirements specified in sections 1.3.1-1.3.2 and to reconfigure the mill to changeover product specification. An estimate of the manpower required and associated job functions has to be included in the bid.

It is required that the whole equipment has to be operated by maximum 1 person to operate in full range of production. An estimate associated job functions has to be included in the bid.

1.4	SCOPE OF SUPPLY

1.4.1	General

The purpose of the following specification is to select a supplier in the scope of design / delivery - based on DDP Kraków - in line NDT Flux leakage system for processing pipes of Ø76,1– Ø168,3mm OD tube, as well as all related devices, so-called turnkey delivery.

On  Buyer’s will remain the execution of construction (foundation) works, preparation of the area for equipment and supply of the necessary utilities (electricity, water, compressed air) to previously established connection points.

It is the intent of these specifications to identify the minimum quantities and quality of mechanical and electrical equipment and systems which will be a part of complete Pilot Line. Compliance with the set minimum does not waive the Bidder's responsibility to provide a fully co-ordinated operating drive system consistent with the functions offered by him.

Machinery will be well designed and soundly constructed. All materials and workmanship has to be first class in every respect and conform to the latest shop practice of recognized reputable manufacturers of equipment of the various types required.

The Bidder has to include with the bid, a listing of the suppliers of  electrical, mechanical, hydraulic, pneumatic components, i.e. gearboxes, couplings, bearings, cylinders, etc.

The mechanical and electrical equipment and systems design has to be suitable for operation in a heavy duty steel mill environment operating 21 (twenty-one) turns/shifts (8 hrs per shift) per week, 50 (fifty) weeks per year. Annual production requirements are based on 15 (fifteen) turns/shifts per week, 48 (forty eight) weeks per year.

CE marking is mandatory for all equipment. All components delivered directly or by subcontractors must be CE marked.

The equipment supplied has to include all necessary guarding, interlocking, lockout and isolation features (electrical and mechanical) and equipment to optimize mill operation and maintenance. The equipment has to meet the requirements of the Machinery Directive 2006/42/EC.

All electrical equipment has to be design and execute considering IP 54 protection.

Equipment has to be cabled and piped from control/execution elements to junction boxes or pick-up points including required filters.

The offer has to include the set of final length-cut and numbered electrical cables to connect the machines to electrical cabinets and control desk/panels. Cables will be delivered with the equipment already connected on one side.

All equipment has to be winterized - pneumatics electro valves and filtering to be contained in heated boxes, hydraulics tanks has to be equipped with heaters or recirculation operated modus, emulsion with recirculation operated modus.

The offer has to include necessary integration of the tube mill system such as but not limited to material flow and electrical control for the associated equipment. The Bidder in cooperation with suppliers of other components of the production line will ensure the correct integration of devices.
The production line will consist of the following devices:

- Tube mill

- NDT weld testing system

- NDT full body testing (flux leakage system)

- ID scarfing system

- High frequency welding system

- ID blowing and chamfering equipment

- Automatic packaging machine

- In line marking equipment

- OD measuring system

Sections 1.4.2 show the Buyer's requirements for devices being the subject of this specification.

1.4.2	Specific

## Equipment to be installed “In LINE”, after pipe cooling, between last calibration stand and first turk-head and cut off, on an area where the pipe is round.

Production line forming section method is constant center height.

The equipment is intended for tubes body (full body) inspection as required by technical standards and specifications referenced in this documentation.

The equipment should be able to detects longitudinal oriented imperfections according to EN ISO 10893-3 on the entire tubes body surface (full body). In addition, the device should be able to carry out the electromagnetic leak-tightness test according to EN ISO 10893-1.

The Flux leakage system must be additionally supported with additional NDT eddy current probe for weld testing. The eddy current probe will be installed after tube welding, before cooling system.

The ET detector should be combined with flux leakage system and should open the rotary testing head to secure the electronics and sensitive part of the detection system from mechanical damage caused by open welds.

## Technical preference: 	in the sizing area.

Detected defects 		100%

Defects to be identified	Longitudinal 
Acceptance level		min. F2 according to EN ISO 10893-3

Power supply			variable DC power

Paint marking 			to identify the flaws area

Sorting: 			two classes "accepted" / "rejected"

Production scope: 	round pipes 76,1 – 168,3 mm with wall thickness 2,0– 6,3mm

- System has to include, but not limited to:

- Magnetizing unit – for magnetic technology

- Coil/Semi-coil or Probe support – adapted to mill structure - with engagement system

- External calibration structure

- NDT eddy current probe with all auxiliaries,

- Engaging power system (hydraulic station, pneumatic or mechanical)

- Main control panel in the main control cabin of the line.

- Additional control panel near the equipment.

- Set-up stand

- Tools for maintenance, set-up and size change

System has to be also able to transport through the whole device square and rectangular hollow sections produced on the line. Size range from 60x60 – 120x120 square and 80x40 – 160x80 rectangular. Squares and rectangular shall not be tested.

The equipment shall be integrated in pipe mill automation.

All necessary testing parts (shoes, coils, semi-coils, probes, adapters etc.) have to be included in supply.

1.5	SPARES

The Bidder has to submit at the bid stage the list of recommended mechanical and electrical spares and pricing in two separate categories as follows:

- Operational spares

- Consumable spares (2 year’s operation)

- Start-up spares

Spares pricing must be valid throughout the life of the contract.

1.6	SERVICES

1.6.1	Turn-key Installation

All below listed works will be provided by the Bidder with his workforce and  supervision included.

The turnkey installation offer must include the following:

- Equipment unpack, position on foundation, pre-alignment.

- Performing all assembly, erection and installation works, resulting from the documentation prepared by the Contractor.

- Electrical cabling and connection;

- Hydraulic piping & connection

- Pneumatic piping & connection

- Assembly, installation and erection of equipment.

- Checking for tolerance, accuracy and all tests of the equipment as provided in the Contract.

- Commissioning of installed equipment, with provided detailed commissioning manuals in Polish and English language.

- Any other work provided for in the Contract.

The Buyer will ensure crane service including crane operator for all installation works if required by the Bidder.

The offer must include the presence of a coordinator with the required authorizations to perform the subject of the contract in terms supervising the proper conduct of work in all it’s aspects and all stages of implementation.

The coordintator will be also responsible of of Health & Safety services of the works carried out by the contractor and/or it’s subcontractors (during the working hours of the contractor’s employees and/or it’s subcontractors). All works has to be done in accordance with ArcelorMittal Poland Health & Safety rules.

1.6.2	Training

The Bidder has to instruct the Buyer's personnel in the proper operation and maintenance of the Bidder-supplied equipment including Subcontractor-supplied equipment.

The bidder has to assume minimum 5 working days time for Buyer personnel training.

Training sessions has to be conducted in small groups and the Bidder has to supply all required manuals and instruction materials in Polish and English language. The trainings will be conducted in Polish language.

The offer must include the preparation and conduct of three separate and dedicated training programs for:

a.	Operating personnel - 4

b.	Maintenance personnel - 6

c.	Engineering/Maintenance technology - 1

The end result of these programs has to be that the trainees have developed sufficient understanding of the facilities that they can operate, maintain and design future enhancements for the equipment.

Electrical training has to include field instruction covering PLC, drives, rectifiers and other control systems application.

Engineering/Maintenance technology programs has to include in-depth training sufficient to allow personnel to fully understand the design and operation of the control systems for design of future enhancements.

Mechanical training has to include maintenance and operation of all mechanical/auxiliary systems.

Hydraulic training has to include maintenance and operation of the complete hydraulic system.

Operation training has to include:

Operating practices and their implementation instructing the trainee how to start-up the equipment and monitor and control it in normal operating conditions and to shut down the equipment.

Operating practices reviewing abnormal and alarm condition procedures.

Other operating practices including troubleshooting and diagnostics, emergency procedures and routine inspection and maintenance.

2.0	EQUIPMENT SPECIFICATIONS

2.1	GENERAL

a.	The Bidder has to provide all necessary tools and devices to change the forms, gear boxes, electric motors, etc. where they are not accessible to the EOT crane and where special lifting devices are required for use with the overhead cranes.

b.	All equipment has to be prime painted and finish painted unless otherwise specified in this Specification.

c.	The maximum average 8 hours work allowable noise level for each piece of equipment has to be 80 dB maximum with maximum temporary peak of 86 dB.  Measurements has to be taken during normal operating conditions with the microphone located 1 meter from the equipment under test.

The Bidder has to make every effort to design and manufacture the equipment to comply with the maximum allowable noise levels.  If there is any equipment that exceeds the maximum allowable noise levels, the Bidder has to identify each piece of equipment and its noise level in his technical proposal.

d.	The Bidder has to ensure that all equipment/process complies with all codes and standards, where applicable, regarding safety (i.e. rotating shaft guarding, barriers, etc.) as per EU regulations.

- All equipment has to be furnished with lifting lugs.  Lifting lugs are to be positioned to assure balance and minimize handling hazards.  Lifting lugs and lifting structures has to be designed to Buyer's lifting and structure design committee rules.

f.	All pipes has to be clamped such that the vibrational velocity is less than sixteen (16) mm/s and vibrational acceleration is less than five (5) g.

- All equipment has to be unitized, pre-assembled and tested in the Supplier's Shop as much as possible to minimize field installation.  Equipment units has to be complete at Supplier's Shop with all attached piping, fittings, valves, gauges, controls, localized lubrication blocks and cylinders installed and tested.  All equipment units and components to be tested for 1.5 times the design pressure.  If dismantling equipment into sections for shipping, list overall size and height, and extent of re-assembly required.

- Valve stands has to be prewired. The Bidder has to consider other electrical unitization where such unitization will contribute to minimizing field installation.  Remote I/O has to be used wherever practical.

- The Bidder has to include any special tools and devices necessary for maintenance.

- Bidder have to use exclusively SI(meter) measurement system

2.2	MECHANICAL SPECIFICATION.

a.	All weldments must be stress relieved prior to machining.

b.	Coupling shrink fits will be .0005” per inch of shaft sixe to a maximum .002”.

c.	Align coupling faces .0005: per inch of face to a maximum .0025.

d.	Coupling diametrical runout to be within .003”.

e.	Coupling angular misalignment to be within .0005” per inch of hub face.

f.	Coupling offset misalignment to be no more than .002”.

g.	Minimum gearbox service factor to be 1.25.

h.	Flatness tolerance per inch of machined surfaces as follows:

| Over | to and including | INCHES |
| --- | --- | --- |
| 0 | 12 | .001 |
| 12 | 24 | .002 |

i.	Machine surfaces so that they are perpendicular within .0005” per foot of the shorter surface to a maximum .003 inches.

j.	All surfaces to be set on grout to be paint and scale free.

k.	Avoid sharp corners and undercuts on all shafts and axles.

l.	Break all sharp edges of machined parts.

m.	All high speed spindles to be balanced in assembly (flying saw).

n.	All manual shutoff valves for air, hydraulic, water, and emulsion to be lockable by a standard safety lock.

o.	All valves and piping for air, hydraulic, water, and emulsion to be labeled as to content and direction of flow.

p.	All hydraulic and pneumatic components has to contain CE marking.

2.3	ELECTRICAL SPECIFICATION

a.	Equipment Standards and Special Approvals

It has to be the Bidder’s responsibility to ensure that all electrical equipment supplied, either directly or via sub-suppliers, meets all EU codes and standards. Where special approval is required it has to be arranged and obtained by the Bidder prior to shipment of equipment to Buyer’s site.  All associated costs has to be borne by the Bidder.

All measurement devices must have EU Metrology approval and to be according Polish Legislation.

b.	Co-ordination and System Integration

It has to be the Bidder’s responsibility to provide coordination and integration of all electrical systems and sub-systems supplied by the Bidder directly or via sub-suppliers.  In addition, the Bidder has to work with the Buyer to integrate components or systems supplied by the Buyer to provide a fully integrated facility.

c.	All electrical components has to contain CE marking.

d.	Incoming Power Supply

The Buyer has to supply in sufficient quantities electrical power at the following voltages:

- Process Power:

- 400 Volts, Three Phase, 50 Hertz

The Bidder has to be responsible for providing suitable transformation to any voltages not listed above.

e.	Equipment Common Features

The following features are common to all electrical equipment supplied by the Bidder:

- Equipment has to be unitized prior to shipment to the maximum extent practical.  This has to include wiring of all pushbuttons, sensors, switches, etc. onto terminal blocks and/or remote I/O mounted in a panel that is attached to the equipment.

- Each operator panel, desk, terminal box and electrical panel has to have a nameplate attached  in a clearly visible location which includes the following information:

- Functional Descriptor,

- Serial no.

- Operating voltage, # of Phases, Hertz

- Rated ampacity

- Maximum Interrupting Capacity (amps)

- Each field device, i.e. solenoid valve, limit switch, sensor, etc. has to have a label on or near the device indicating its function and associated I/O point with link to the electrical documentation, e.g. Peeler Table Raise O1/11.

- Every wire  not interposied by terminal blocks has to have its own identification number. Wire numbers has to change across terminal blocks.

- Wires has to be identified as follows:

- Power wiring has to be identified by the sheet/line number of the associated electrical elementary diagram

- I/O wiring has to be identified by the Rack and I/O point.

Each panel containing remote or local I/O has to include a complement of 10% spares.

Each operator station or desk has to have holes punched and capped for additional operator devices (equiv. 10% spare).

f.	Programming Software

- The Bidder has to provide one (1) licensed copy of each type of software as follows:

- PLC programming software

- AC/DC Drive configuration/tuning software

- HMI programming/configuration software

- Other software required to configure/maintain the facility

- In addition, the Bidder has to supply any hand held programming/configuration devices that may be required.

g.	Motors

All motors has to have the following features:

- AC Motors -- auxiliary and process (fixed speed)

| Voltage | 400 volts 3 phase 50 hertz |
| --- | --- |
| Service Factor | 1.0 |
| Design | IEC |
| Enclosure | TEFC |

- AC Motors -- process (variable speed)

| Voltage | As required by drive supplier |
| --- | --- |
| Service Factor | 1.5 |
| Design | IEC (unless otherwise required by drive supplier |
| Enclosure | TEFC |

- Preferable AC motors

- Note:  Motors designed for continuous operation within the specified speed range without injurious heating

h.	Sensors and Detectors

All sensors and detectors required for line control and interlock has to be provided.  All limit switches has to be of heavy-duty, dust proof and waterproof construction and be selected to suit the application and environment.  All logic and sequencing has to be designed to fail-safe.

2.4	EQUIPMENT CONTROL

a.	Control equipment has to be designed and arranged to minimize field installation.  Remote I/O and/or intelligent I/O modules has to be used wherever practical.

b.	The equipment control has to be comprised of a complement of PLC’s including software, variable speed and fixed speed drives, operator stations, and associated auxiliary equipment.  The system has to be fully integrated which includes communication between all PLC’s, drives, auxiliary equipment, and HMI devices.  Additionally, the Bidder has to supply and install an appropriate interface to permit communication (Ethernet TCP/IP) with a Level 2 computer system (see Section 2.5).  The control has to be coordinated to allow independent and synchronized operation of each section of the mill.

c.	The Bidder has to incorporate a hardwired emergency stop system which includes:

- Illuminated mushroom head pushbuttons placed on all operator stations and other locations where operator intervention may be required (to be discussed).

- Provision for light curtains located as appropriate.

d.	The Bidder has to supply the operator stations

e.	An HMI has to be provided at the main operator desk that has to incorporate equipment fault annunciation as well as other graphic and visual indication to facilitate equipment operation.  Control and monitoring at the main operator desk has to include, but not be limited to:

- Monitoring of drive signals (current, speed, etc.)

- Selection of modes

- Setting of manual references

- Control of drive contractors

- E-stop

- Data entry, collection and reporting

2.5	LEVEL 2 COMPUTER INTERFACE REQUIREMENTS

a.	The Buyer intends to implement a Supervisory Control and Data Acquisition (SCADA) system with the tube mill.  The SCADA system will coordinate the distribution of manufacturing plan data to the tube mill equipment, will insure traceability of material and will provide a central data store for material quality and equipment performance data.

b.	In order to perform these functions the Buyer requires that the equipment be capable of the following general functions:

- Communications with the SCADA system using Ethernet TCP/IP protocols

- Provide material movement data to the SCADA system

- Provide all material quality sensor raw data to the SCADA system.

- Provide equipment status information to the SCADA system

- SCADA supplied or operator supplied setpoints to be operator selectable on a function by function basis

3.0	SUPPLEMENTAL INSTRUCTIONS TO BIDDERS

3.1	Delivery of Equipment

The offer has to include delivery to the Purcharser site – means DDP Kraków.

a.	The equipment or parts of equipment has to be suitably prepared and packed for shipment.  The weight and unit dimensions for each of the packages has to take into account the requirements of any special conditions of shipment, unloading and transportation for delivery to the Buyer.

b.	The equipment has to be so packed and protected as to prevent any damage or deterioration under usual transport conditions with proper anti-corrosion and/or anti-rust compounds or coatings, protective waterproof wrapping and/or packing as the case may require.

c.	When bolts or piping fittings, etc. are shipped, different kinds and sizes has to be kept separate either by separate containers or partitions in the same container.

d.	Machinery and equipment has to be painted according to the Contract.  All mechanical surfaces has to be adequately protected against the elements. Surfaces which are exposed and liable to mechanical damage has to be further protected by wooden packaging.

e.	Finished surfaces of machinery and equipment has to be thoroughly coated with a protective rust preventive material that can be readily removed.

f.	The Bidder has to be held solely responsible for any damage, loss or deterioration to the equipment caused by inadequate protection or packing.

g.	Each separate package has to have the following information stencilled on it:

- The Buyer’s Project name and number

- The Buyer’s Purchase Order number

- Package Number using a consecutive system

- Storage requirements indicating:

- Inside or outside storage

- Which end has to be stored up

- Any other special storage requirements

h.	All individual parts of equipment or spares whether shipped loose or in packages has to be equipped with labels or stencils identifying the following:

- Package number

- Individual part number

- Equipment number to which the part belongs

- Equipment assembly drawing number

i.	A detailed packing list has to be provided for each package and be identified by the package number.  One copy of the packing list has to be sent by Telefax to The Buyer at time of shipment with a second copy attached to the package in a clear plastic envelop.

j.	It is the Buyer's requirement to have equipment pre-assembled to the maximum extent permitted by shipping facilities to minimize field erection and assembly time.  The Bidder has to clearly state in his technical proposal the extent of disassembly for shipment and the degree of interconnection required for erection.

3.2	DOCUMENTS TO BE SUPPLIED WITH THE BID

The following specification applies to documentation that the Bidder will provide together with the technical offer. The following documentation is required by the Buyer in order to obtain a building permit and carry out preparatory work on the Buyer's side.

All documents has to be delivered in metric dimensional standard.

All documents has to be in both Polish and English language.

    - General

- The bidder has to include with his bid description all necessary manpower required for equipment operation (operators – per shift, maintenance electrical and mechanical – per shift, size changing – per day, crane operators – per shift, transfer bundles to warehouse – per shift etc).

3.2.2	Mechanical/Civil documentation

  - Line layout drawing.

  - General arrangement drawings of equipment including size, footprint, trenches, pits, etc.

  - DOCUMENTS TO BE SUPPLIED AFTER A CONTRACT

The following specification applies to documentation that the Bidder will provide no later than 2 months after signing the contract. The submitted documentation must be approved by the Buyer to start the next stage of the project.

All documents has to be delivered in metric dimensional standard.

All documents has to be in both Polish and English language.

3.3.1	Mechanical/Civil documentation

The Bidder submission has to include but not be limited to the following documents and drawings.

For Approval; the Bidder has to submit three (3) prints of all drawings.  As Built; Diskettes has to be submitted with drawings saved as AutoCAD .dwg or .dxf format.  Three (3) copies of all manuals and specifications has to be submitted.

a.	As Sold specification and As Built specification.

- Layout drawings of the line.

c.	Assembly drawings of all equipment.

d.	Piping schematics, instrumentation and utility flow diagrams:

- water system

- pneumatic systems

- grease lubrication systems

- lubrication oil systems

- hydraulic systems

- drainage systems

e.	Equipment detail drawings complete with material specifications and bills of material.

f.	Operating and maintenance manuals including:

- drawing index

- general arrangements of all major assemblies and system schematics

- operating instructions describing function, sequential operation and control, design criteria and limits

- maintenance requirements and frequencies

- troubleshooting

- manufacturer's listing including description of item, application information, drawings, standard data sheets, etc.

- lubrication instruction

- maintenance manuals supplied are to include catalogues, engineering information and drawings sufficient in detail and identification for ordering of spare parts; manuals are to be prepared specifically for the equipment supplied, i.e. general manuals for similar equipment are not acceptable.

g.	Installation manuals including:

- complete equipment weight list of each piece as delivered indicating shipping splits

- complete bills of material for installation

- equipment alignment tolerance

- detail procedure of erection work and piping work

- grouting specification

h.	Commissioning manual including:

- detail schedule and procedure of commissioning

- measuring and calibrating apparatus

- personnel organization

- schedule for preparing materials supplied by the Buyer (e.g. test coils)

i.	Description of required utilities including:

- required quantity for each utility

- required specification for each utility

j.	Documents and correspondence required for all necessary authority approvals.

k.	All construction drawings and bills of material for installation of equipment and piping.

l.	Shop test procedures and certificates.

m.	Foundation drawings showing – if applicable:

- load distribution

- elevation and configuration

- piping pit, cable pit, cable conduit

- embedded steel

- stationary floor plates, staircases, etc.

n.	Spare parts lists – mechanical, hydraulic, lubrication

o.	Detailed construction drawings for easy wear items – consumable within year.

3.3.2	Electrical documentation

The Bidder has to submit but not be limited to the following documents and drawings.

For Approval, the Bidder has to submit three (3) prints of all drawings.  As Built; CD’s has to be submitted with drawings saved as either AutoCAD .dwg or .dxf format.  Three (3) copies of all manuals and three (3) copies of all specifications has to be submitted.

a.	As Sold specification and As Built specification.

b.	Single line diagrams.

c.	Outline drawings of equipment.

d.	Test record sheets.

e.	Spare parts lists.

f.	Specification and manuals for operation, control and monitoring of the following:

- control and/or operation flow diagrams

- schematic diagrams of the control systems

- documents on operation and control methods

- control and monitoring devices

- layout of the desks and operator panels

- drawings of the MMI graphic displays

- interlock lists

- sensor and detector lists

- solenoid valve lists

- motor winding and bearing data

g.	Software documents of PLC's and MMI's.

h.	Hardwired sequence diagrams.

i.	Instruction and maintenance manuals.  Maintenance manuals supplied are to include catalogues, engineering information and drawings sufficient in detail and identification for ordering of spare parts.  Manuals are to be prepared specifically for the equipment supplied, ie. general manuals for similar equipment are not acceptable.

j.	Design drawings and calculation sheets.

k.	Description of required utilities including:

- required quantity for each utility

- required specification for each utility

l.	Documents required for authority approvals.

m.	Installation manuals including:

- complete equipment weight lists of each piece as delivered

- complete bills of material for installation

- tolerance of each equipment alignment

- detail procedure of equipment and cable installation

n.	Commissioning manuals including:

- detail schedules and procedures of commissioning

- measuring and calibrating apparatus including detailed specifications

o.	Equipment requisitions

p. 	Measuring protocols required by Polish standards

3.4	WORKS BY BUYER

The following work and equipment will be supplied by the Buyer:

- Removal and relocation of all existing equipment required for new work.

- All building changes to suit installation of new equipment.

- Construction of all foundation work including the supply and installation of all embedded anchor bolts according to Bidder’s drawings.

- Supply and installation of all embedded curb angles, pit and trench cover plates,

- Supply of all media utilities (electricity, compressed air, water) to equipment  as requested by the Bidder in his tender. The Bidder has to provide all required media characteristics and drawings with specified access points. Media distribution between equipment elements has to be done by the Bidder.

- Receive, unload, and inspect all equipment at The Buyer’s site together with a representative of the Bidder.

3.5 DEADLINE FOR PROJECT IMPLEMENTATION

The assessment of the project implementation deadline will be made on the basis of the submitted implementation schedule of the subject of the contract in accordance with the following formula.

4.0	PAYMENT CONDITIONS.

All payments will be payed thirty (30) days end of the month as from the BUYER’s quantitative and qualitative acceptance  of the corresponding contractual events.

| Item no. | Description of the concerned contractual events | Date of contractual events | Contractual price instalments (in % of contractual price) | Remarks (subject to Bank Guarantees or to liquidated damages) |
| --- | --- | --- | --- | --- |
| 1. | Down payment at the order ( subject to the due and prior receipt of the Contractor’s unconditional order acknowledgement issued by the Contractor) | J | 15% | Subject to Bank Guarantee as per the agreed format attached with the contract. This will be valid till the Delivery of the equipment. |
| 2. | Delivery to the Buyer Documentation as per point 3.3 of Technical Specification | J + 2 months | 15% |  |
| 3. | Mechanical Inspection at Supplier’s site | J + 8 months | 30% |  |
| 4. | Delivery of equipment | J + 9 months | 10% |  |
| 5. | Industrial commissioning | J + 10 months | 10% |  |
| 6. | Provisional Acceptance Test (PAT) | J + 10 months | 10% |  |
| 7. | Performance Acceptance Certificate (PAC) | J + 13 months | 10% | Subject to Bank Guarantee as per the format attached in the contract. This will be valid for 12 months from successful PAC signoff. |

4.1. Provisional Acceptance Test.

Provisional Acceptance Test (PAT) will be carried out after all devices have been properly installed and commissioned.
A positive result of the Provisional Acceptance Test (PAT) is the basis for issuing the Performance Acceptance Certificate.
A report on the successful completion of the Provisional Acceptance Test (PAT) will be signed by both parties.

Provisional Acceptance Test will be carried out on a 3 dimensions of pipes in 3 wall thickness combination :

| OD/WT | 2,9 | 4,0 | 6,0 |
| --- | --- | --- | --- |
| 76,1 | 70 | 50 | 45 |
| 114,3 | 50 | 45 | 35 |
| 168,3 | - | 35 | 25 |

- Provided speed in pcs/min is minimum required to perform and be accepted. Up to 5% deviation is acceptable.

- System has to ensure detection of longitudinal imperfections on outside and inside surface imperfections of pipes of the line size range.

- System has to be able to detect imperfections on pipes produced from black steel stip as well as with metallic and galvanic coatings with coating thickness up to 35 μm – one side and 70 μm both sides.

- The system has to be able to detect defects on the pipes from production range of the line with the quality F2 acc. to EN 10893-3 with speed of the line as above.

- Equipment has to detect defected pipes and give signal to effective rejection and separation to collecting bin for rejected pipes.

- The system has to test pipes with tolerances of OD, wall thickness, ovality and straightness as shown in table 1.3.1.

- Confirmation of successful PAT will be detection of longitudinal defects of depth within 5% of wall thickness or minimum 0,3mm, width max 1mm, length max 25mm.

- Calibration of equipment has to be done as per EN 10893-3 on reference pipes prepared as per EN 10893-3.

- Provisional Acceptance Test will be carried out in minimum 1 hour campaign for each item.

- The test campaign can be counted as successful on in condition of continuous production, without stoppages and breakdowns.

- Tubes produced during PAT has to meet quality requirements of the material specification standards and any other requirements from the contract.

- During the conduct of the trial PAT the equipment is working in fully automatic way, without operator’s help under regular conditions.

- In case of defeat, re-tests will be carried out after three days. Maximum may be carried out two further trials.

4.2. Performance Acceptance Certificate.

Perfomance Acceptance Certificate (PAC) will be issued by the Buyer and signed by both parties three months after the installation, commissioning and start-up of all devices.
During the trial period, the equipment must operate without failures and stops, and all performance parameters specified in the Provisional Acceptance Test (PAT) must be maintained.